Section 4, Page 1 – REVISED February 2003

 


4.0
System Component Failure Prevention and Emergency Supplies
The preventions listed below have been established for the various locations of concern with regard to spills.

4.1 Tailings System

· Couplings are inspected and replaced if required 

· The entire length of the line is inspected weekly

· Dikes are inspected daily during runoff period

· Tailings pumps are on a preventative maintenance system

· Backup tailings pumps are on standby

· Lines are normally flushed with water once every fourteen days

· Dump ponds are cleaned out as required, generally during summer thaw

· Water levels are monitored continuously at East Twin and West Twin Reservoirs

· Water quality within the reclaim pond is monitored monthly and more frequently as elevation rises to the top of the decant structure

· Water elevation in the West Twin Disposal Area Reservoir is controlled by a stop log system which can be raised to lower water levels in emergency situations or as inventories require release of such water

· pH control is maintained within the pond system through reclaim water circulation to the plant and back to tailings

· Metal values in decant waters continually meet effluent standards and all releases of water are monitored daily for flow volume rates and sampled for metal analysis 

· 1 metre freeboard elevation is maintained in the Surface Cell by using decant lines capable of removing 5 – 6 cubic metres of water per minute over the dike structure into the reclaim reservoir

· 2 portable diesel pumps are available for pumping from the Surface Cell into the reservoir each capable of pumping 3 – 4 cubic metres per minute, these pumps can also be employed in the pH adjustment of polishing pond waters with lime addition and mixing prior to release

4.2
Fuel Storage Impoundment

· Diesel fuel is pumped through a solid welded line to the storage tank at the industrial plant 

· Each tank is equipped with level control and alarm 

· Pumps are internally inspected quarterly and overhauled annually

· Pressure sensor will shut down pumps if a leak in a line occurs

· Weekly line inspection

4.2 Fuel Storage Impoundment (cont’d)

· Jet fuel and gasoline are trucked from the impoundment

· All tanks are loaded via automatic shut off valve and clip-on connectors

· All tanks are inspected weekly for potential leaks

· General inspections are conducted daily

· Oil supply company has on-board spill equipment, a requirement by the Canadian Coast Guard

4.3 Chemical Storage Yard

· Process chemicals contained in drums and weather resistant tote bags are stored within the storage yard
· These areas are routinely inspected as chemicals are continually being consumed from storage yard

· No special spill prevention measures are necessary

4.4 Refuelling Depots

· Refuelling occurs at the Main Adit and at the industrial site. These areas are in continuous use and thus routinely inspected

· For remote mine sites, vehicles are refuelled from the tank truck

4.5 Potable Water Supply

· Daily inspection of the diversion channel at East Twin Lake during the runoff period

· Continual water level monitoring

· Monthly sampling and metal analysis











-
NANISIVIK MINE-

Emergency Response Plan

