
DEW LINE CLEANUP:  POST-CONSTRUCTION - LANDFILL MONITORING

VISUAL INSPECTION CHECKLIST

INSPECTION REPORT – PAGE 1 OF 2

	SITE NAME:
	Cambridge Bay Cam M

	LANDFILL DESIGNATION:
	West

	DATE OF INSPECTION:
	August 2005

	DATE OF PREVIOUS INSPECTION:
	August 2004

	INSPECTED BY:
	Alistair Kent

	REPORT PREPARED BY:
	Alistair Kent


VISUAL INSPECTION CHECKLIST – WEST LANDFILL - INSPECTION REPORT – PAGE 2 OF 2

	Checklist Item
	Present

Yes/No
	Location

(Describe relative to existing monuments/features and relative to landfill design i.e. surface, berms, toe)
	Length
	Width
	Depth
	Extent relative to Area of Landfill (%)
	Description
	Photographic Records

Focal length,

location, view point & direction (relative to magnetic north)

Feature of note

Scale
	Additional Comments/Preliminary Stability Assessment

	Settlement
	Yes
	Northern edge 
	8 m
	8 m
	<0.5 m
	< 5%
	Localised cracks around north and west sides of depressed area
	WLF 8 and WLF8A
	Marginal

	Erosion
	Yes
	Appear unchanged from previous year
	30-50 cm
	8-10cm
	12 cm
	<1
	Rills, Erosion, channelling, deposition
	WLF 7
	

	Frost Action
	No
	
	
	
	
	
	
	
	

	Animal Burrows
	No
	
	
	
	
	
	
	
	

	Vegetation
	No
	Sparse across top surface, and minimal on side slopes
	WLF 9
	

	Staining
	No
	
	
	
	
	
	
	
	

	Vegetation Stress
	Yes
	Dead scrub
	
	
	
	
	
	WLF 5
	Appears localised

	Seepage Points
	No
	
	
	
	
	
	
	
	

	Debris Exposed
	Yes
	As previous year
	
	
	
	
	No change
	WLF 3
	Acceptable

	Presence/Condition – Monitoring Instruments
	No
	
	
	
	
	
	
	
	

	Features of Note.
	None
	
	
	
	
	
	
	
	


D:\data\DEW\Corrections january 2006\50714 App D1 West Landfill Visual Inspection Checklist10-Oct-2005.doc



