B1.	NWS Landfill	1

B.1.1	Landfill Summary	1

B.1.2	Visual Inspection	1

B.1.3	Soil Sampling	4

List of Tables

Table B-1	Summary of 2011 Soil Analysis – NWS Landfill	5

List of Figures

Figure B1	NWS Landfill	3

List of Appendices

Appendix B1		Visual Inspection Records

Appendix B2		Site Photographs

# B1.	NWS Landfill

## B.1.1	Landfill Summary

The NWS Landfill is located approximately 700 m west of the main station and encompasses an area of approximately 42,000 m2. Historically at this landfill, waste material was first burned and then covered with gravel.  The depth of landfilled waste materials is approximately 0.5 m. The landfill configuration, including photograph and sample locations, is shown on Figure B1. Prior to the remedial work in 2004, DCC had classified this landfill as a moderate potential environmental risk and areas of DCC Tier II soil were identified. The remedial work for this landfill included the excavation of known contaminated surficial soils and regrading with additional granular fill.  The cover of the landfill has sparse vegetation. The surface consists of a veneer of pebbles and cobbles overlying the granular and silt cover.

For 2011, the monitoring requirements for the NWS Landfill included a visual inspection and soil sampling and analyses.

## B.1.2	Visual Inspection

Based on the 2011 visual inspection, the NWS Landfill area appears to be in good condition and does not exhibit signs of imminent slope instability or final cover failure. There were no changes compared to the 2005 visual inspection and the overall landfill performance is considered to be “acceptable”. Appendix F1 presents a summary of the 2011 visual inspection results and features of note are shown on Figure F1.

There was one minor erosion feature noted in 2005.  In 2011, this feature (Feature A) was again observed; however, it was interpreted to be a scour mark cause by an adjacent boulder likely being pushed across the ground surface by heavy machinery.

An area of ponded water between the central and eastern landfill areas was observed in 2005 and 2011 (Feature B).  The ponded water was approximately 3 m long and 3 m wide.  The water did not have a visible sheen.

Figure B1	NWS Landfill

## B.1.3	Soil Sampling

Soil samples were collected at the designated locations of P3-1, P3-2, P3-3 and P3-4.  The sampling locations are shown on Figure B1. A photograph of each test pit for each location sampled is shown in Appendix B2.  Soil samples were analyzed for DCC metals, total petroleum hydrocarbons (TPHs) and polychlorinated biphenyls (PCBs).  Analytical results and depths of samples are provided in Table B-1 and the Laboratory Reports are provided in Appendix G.

TPHs were detected in samples collected from the four locations.  The greatest concentration of TPH was 1612 mg/kg at P3-4 (0 m to 0.15 m).  The analytical results for metals and PCBs in soil samples were less than the DCC DEW Line Cleanup Criteria for soil (DCC Criteria).

Table B-1	Summary of 2011 Soil Analysis – NWS Landfill

Appendix B Attachments

B1	Visual Inspection Records

B2	Site Photographs

APPENDIX B1

Visual Inspection Records

Appendix B2

Site Photographs