F1.	NORTH Landfill	1

F.1.1	Landfill Summary	1

F.1.2	Visual Inspection	1

F.1.3	Soil Sampling	3

List of Tables

Table F-1	Summary of 2011 Soil Analysis – North Landfill	4

List of Figures

Figure F1	North Landfill	2

List of Appendices

Appendix F1	Visual Inspection Records

Appendix F2	Site Photographs

# F1.	NORTH Landfill

## F.1.1	Landfill Summary

The North Landfill is located approximately 650 m north-norhtwest of the main station and encompasses an area of approximately 28,000 m2. The landfill is comprised of three lobes.  The depth of landfilled waste materials is approximately 1 m. The landfill configuration, including photograph and sample locations, is shown on Figure F1.  DCC classified this landfill as a low potential environmental risk and remediation of this landfill included removal of scattered surface debris and regrading with additional granular fill.  The cover of the landfill has no vegetation. The surface consists of a veneer of pebbles and cobbles overlying the granular and silt cover.

For 2011, the monitoring requirements for the North Landfill included a visual inspection and soil sampling and analyses.

## F.1.2	Visual Inspection

Based on the 2011 visual inspection, the NWS Landfill area appears to be in good condition and does not exhibit signs of imminent slope instability or final cover failure. There were no changes compared to the 2005 visual inspection and the overall landfill performance is considered to be “acceptable”. Appendix F1 presents a summary of the 2011 visual inspection results.

There were no erosion, settlement, frost action, burrowing, vegetative stress, seepage/ponded water, staining or debris features observed in 2005 or 2011.

Figure F1	North Landfill

## F.1.3	Soil Sampling

Soil samples were collected at the designated locations of P3-12, P3-13, P3-14, P3-15 and P3-16.  The sampling locations are shown on Figure F1. A photograph of each test pit for each location sampled is shown in Appendix F2.  Soil samples were analyzed for DCC metals, total petroleum hydrocarbons (TPHs) and polychlorinated biphenyls (PCBs).  Analytical results and depths of samples are provided in Table F-1 and the Laboratory Reports are provided in Appendix G.

TPHs were detected in samples collected from the five locations.  The greatest concentration of TPH was 312 mg/kg at P3-14 (0 m to 0.15 m).  The analytical results for metals and PCBs in soil samples were less than the DCC DEW Line Cleanup Criteria for soil (DCC Criteria).

Table F-1	Summary of 2011 Soil Analysis – North Landfill

Appendix F Attachments

F1	Visual Inspection Records

F2	Site Photographs

APPENDIX F1

Visual Inspection Records

Appendix F2

Site Photographs