Water Licence Application 
                            Coral Harbour Dev Corp            
Initial Technical Review      


                   15 November 2002




Application Form - Summary of Information Submitted 

The application that was submitted on 29 October 2002 contained maps indicating the locations of the proposed temporary camp and the greywater disposal sump, as well as proposed water supply for the camp. However, this information is somewhat rudimentary, and while it provides certain topographic information helpful in assessing drainage patterns, information pertaining to specific drainage features or watercourses on the proposed site is somewhat lacking, due to the coarse scale of the map provided.  
Spill Contingency Plan:

As this is temporary field camp of limited duration, it can be reasonably expected that there will be relatively small amounts of fuel (gasoline and diesel fuel) that will be stored on the site.  The Remote Camp Questionnaire indicates that the total heating fuel storage will be 20 barrels (200L), as well as a 3000 L tank for the storage of gasoline to be used in snowmobiles during harvesting activities. The Board generally requires that all Licensees prepare comprehensive Spill Contingency Plans to establish a state of readiness which will ensure prompt and effective response to possible spills or system failure events. The site-specific contingency plan will assist the Licensee in responding to emergencies such that the impacts to water in particular and the environment and public health in general are minimized.  NWB staff members are presently liaising with Dev Corp staff, to ensure that an appropriate spill contingency plan will be developed for the site.  
Supplementary Questionnaire 

Section II
Attachments

Comment 1. 
Application does not include the general location and layout of the proposed greywater disposal sump; detail is lacking in several areas.   Information is required on the details of construction:

Section III
Water Supply

Comment 1.
Daily requirements, for both human consumption and abattoir operation, have been quoted at approximately 2000 l per day.  It is assumed that this amount will be insignificant in terms of the total volume of the un-named lake which will be utilized for the camp water supply.  Total time at the cam will be 120 days, so the maximum total volume removed from the lake will be approximately 240,000 L.  
Water Treatment

Comment 1.
The Applicant indicates in their application that the water will not be chlorinated.  Therefore, some concerns regarding the potability of the water supply do exist, particularly in view of the proximity of the greywater disposal sump to the water supply.  It is important that a water sampling program be carried out, to confirm that quality meets the Canadian Drinking Water Quality Standards.  The Coral Harbour Dev Corps should closely co-operate with the local Health Centre to facilitate water sampling.  
· What current monitoring data is available to confirm that the water source meet the Canadian Drinking Water Quality Standards?
Water Use and Distribution

Comment 1. 
 The Applicant states that water will be removed from the lake by bucket, so no concerns regarding the entrainment of fish exist.
Section IV
Sewage Disposal

Comment 1:
The Applicant indicates in their application that sewage will be collected in Honey Bags and backhauled to the Hamlet of Coral Harbour disposal facility.  Greywater from the camp and abattoir operations will be disposed of in a sump.  The Applicant indicated this sump will be located a minimum of 50 meters from watercourse.     
· What are the details of the greywater sump?  How will the sump be managed after closure of the camp?
· What is the state of the development of the Abandonment and Restoration Plan for the site?  What monitoring program will be carried out at the abandoned site?   

Section V
Solid Waste Disposal
Comment 1:  The Applicant has indicated that all solid waste will be backhauled to the Solid Waste Disposal Facility operated by the Hamlet of Coral Harbour.  
Section VI
Inspection and Monitoring

Comment 1:
Identify the location of spill kits located within the camp facility on a map or itemize their location. 
· What monitoring program is in place at present to examine the quality of the proposed water supply for the proposed camp? What results are available, and from what source?  

 Section VIII
Public Health

Comment 1:  Only potential concern at this time wrt Public Health is the monitoring program on the water supply, due to the potential impact of the greywater sump and abattoir operations on the water supply for the camp.
Section IX
Technical Information
Comment 1:  No applicable comments at this time.
Any questions with respect to this review can be directed to Jim Wall, Technical Advisor NWB at 867-360-6338 or fax 360-6369 or email nwbtech@polarnet.ca 

PAGE  
\\Mother\users\CENTRAL\Licenses\NWB4-other\Cor-Dev Corp\Technical Review\111502CorDevCorpstechrevapplication-ITAE.doc
Tech Adv
Page 2
17/11/2002

