| Open Burn |  |  |  |
| --- | --- | --- | --- |
| Document Number: | SITE.SOP-5-5020 | Date Prepared: | September 13, 2020 |
| Current Revision No: | 1 | Revision Date: |  |
| Site Location(s) Used: | Goose Property |  |  |

PART 1: Risk Mitigation

A Job Hazard Analysis (JHA) should be developed for a task where a higher level of detail around hazards and controls are required. If a JHA has been developed for this procedure, it should be attached and reviewed as part of the procedure sign-off process.

Associated JHA No.: N/A

| Critical Risk Controls: | ☒ CRC 1 Light Vehicles and Mobile Equipment ☐ CRC 3 Energy Isolation ☐ CRC 5 Lifting Operations ☐ CRC 7 Excavations and Ground Support ☐ CRC 9 Explosives and Blasting | ☐ CRC 2 Equipment Safeguarding ☐ CRC 4 Working at Heights ☐ CRC 6 Confined Spaces ☒ CRC 8 Hazardous Materials ☐ CRC 10 Aviation |
| --- | --- | --- |
| Tools / Equipment: | Burn Cage, Radio, Fire Extinguisher, Shovels, and Rake |  |
| Potential Hazards: | Combustible Material, Heat, Heavy Lifting, Animals, Weather, and Ground Conditions |  |
| Specialty PPE: | Fire Retardant Clothing as Required |  |
| Training Required: | WHMIS, Fit Tested, and Fire Extinguisher Use |  |

PART 2: Work Instruction

| Special Precautions: | Workers are required to ensure that weather factors will not affect open burns. |
| --- | --- |

- Prior to starting open cage burn workers are required to scan work area for bears, wolverines, and wolves.

- Workers will complete a Hazard Assessment prior to starting task.

- Worker will don fire retardant coveralls as required.

- Worker to ensure only suitable waste is placed in open burn cage. Paper products, cardboard and non-treated wood are only permissible materials to be burnt in open burns.

- Worker to ensure products to be burned remain as dry as possible before starting burn process.

- Burns are to take place during favorable weather. (Minimal winds or rain)

- The size of the burn pile will be minimized, and a “hot burn” maintained to create an efficient burn using consistent/constant feed rates to avoid over-feeding and damping the fire

- Record date, time, waste type, waste weight included in open burn

- Initiate burn and oversee progress of each burn.  The Waste Management Employee and designated assistants will not leave the open burn once initiated.

- Fires will be completely extinguished by raking to ensure that any smoldering of material does not persist and material cools.

- Prior to removing ash worker will don respirator equipment following the Respirator Selection and Use SOP. Respirator is always REQUIRED to be worn while working with open ash.

- Once ash is cooled it will be transferred into a 45-gallon drum that is stored on the pallet and covered with a steel lid.

- Once the drum is full the worker will seal the drum with a plastic barrier and tape securely in place. Once there are 4 drums full, sealed and labeled they are to be banded together on the pallet for transport.

- A qualified equipment operator will then come and pick up the pallet of ash and bring to the scale to be weighed.

- Operator will then label the total weight of all 4 drums, pallets, and ash on one of the drums with the final weight.

- Operator will then transport ash to air strip and place in designated storage area for shipping to BBE.

PART 3: Revision History

| Revision Date | Revision No. | Revision By | Purpose for Revision |
| --- | --- | --- | --- |