

Arviat



Baker Lake



Chesterfield Inlet



Coral Harbour



Naujaat



Rankin Inlet



Whale Cove

To:



Arviat



Baker Lake



Chesterfield Inlet



Coral Harbour



Naujaat



Rankin Inlet



Whale Cove

Robert Hunter

Licensing Administrator

Nunavut Water Board

From: Luis Manzo, Director of Lands, Kivalliq Inuit Association

Date: June 23, 2023

Re: Review of Agnico Eagle Mines Limited’s Meadowbank Complex 2022 Annual Report; NWB Water Licences 2AM-MEA1530 & 2AM-WTP1830

# Introduction

The Kivalliq Inuit Association (KivIA) has conducted a review of the Agnico Eagle Mines Ltd. (Agnico Eagle) 2022 Annual Report for the Meadowbank Complex Gold Project, including both the Meadowbank and Whale Tail sites. Agnico Eagle’s submission consisted of the Meadowbank Complex 2022 Annual Report (April 2022) supported by 56 appendices. These documents were submitted by to address requirements within the water licences 2AM-MEA1530 & 2AM-WTP1830.

KivIA has completed this review with the support of the following consultants:

- Prairie Scientific Inc. (PSI; Matt McDougall), aquatic environment specialists; and

- GeoVector Management Inc. (GeoVector; Alan Sexton), geoscience specialist.

# Technical Review

## 2.1 Aquatic Environment Technical Comments (reviewer: PSI)

Comment No. KivIA 1: Pit Lake Conductivity Profiles

Reference: Appendix 12

In the Meadowbank Water Forecasting Update, Agnico Eagle contemplates measuring depth profiles of conductivity in the pits to determine the presence of stratification in the pit lakes. Discussions surrounding the creation of end pit lakes with suitable fish habitat reference the presence of a chemical gradient, with higher concentrations of dissolved solids near the bottom of the end pit lakes. Further information on the presence and stability of stratification in the pits would assist in evaluating the suitability of these lakes for providing fish habitat.

Recommendation: Agnico Eagle should commit to measuring depth profiles of conductivity in the reflooded pits.

Comment No. KivIA 2: Document Control

Reference: Appendix 37

A large number of documents are submitted for review annually. Use of the documents control tables to outline changes in subsequent document versions enable reviewers to efficiently focus their efforts.

Recommendation: Agnico Eagle should ensure the pages and sections modified in subsequent document versions is reflected in the document control table.

## Geoscience Technical Comments (reviewer: GeoVector)

Comment No. KivIA 3: November 28, 2022 29,000 litre fuel spill at km 87 on the AWAR

Reference: Meadowbank Complex, 2022 Annual Report, Table -2; Appendix 6 – Agnico Eagle’s Training Management System and Learning Management System Reports; Appendix 28 – Meadowbank 2022 GN Spills Reports.

Comment: Did the investigation into this significant fuel spill include a review of the training records and maintenance records of mobile equipment for the Inuit Contractors used for hauling fuel? When will AEM determine if this is an insurable event for the Inuit Contractor involved in the spill?

Recommendation: The KivIA would like to see the requested information at AEM’s earliest convenience.

# Closing

The KivIA appreciates the opportunity to provide comments on the 2022 Annual Report for the Meadowbank Complex Project. Please contact Luis Manzo, Director of Lands, (dirlands@kivalliqinuit.ca) should you require more information.

Regards,

Luis Manzo P, Ag.

Director of Lands

Kivalliq Inuit Association

Tel: (867) 645-5731

dirlands@kivalliqinuit.ca