Agnico Eagle Draft in prep for submission as of December 17, 2020

NUNAVUT WATER BOARD

AMENDED WATER LICENCE NO: 2AM-MEL1631

PART A:

# TABLE OF CONTENTS

SCOPE, DEFINITIONS AND ENFORCEMENT	1

- SCOPE	1

- DEFINITIONS	3

- ENFORCEMENT	3

PART B: PART C: PART D: PART E: PART F: PART G: PART H:

PART I: PART J:

GENERAL CONDITIONS	3

CONDITIONS APPLYING TO SECURITY	6

CONDITIONS APPLYING TO CONSTRUCTION	7

CONDITIONS APPLYING TO WATER USE AND MANAGEMENT	11

CONDITIONS APPLYING TO WASTE DISPOSAL AND MANAGEMENT	13

CONDITIONS APPLYING TO MODIFICATIONS	17

CONDITIONS APPLYING TO EMERGENCY RESPONSE AND CONTINGENCY PLANNING	18

CONDITIONS APPLYING TO GENERAL AND AQUATIC EFFECTS MONITORING	19

CONDITIONS APPLYING TO ABANDONMENT, RECLAMATION AND CLOSURE	22

SCHEDULES	25

Schedule A:	Scope, Definitions, and Enforcement	26

Schedule B:	General Conditions	35

Schedule D:	Conditions Applying to Construction	38

Schedule I:	Conditions Applying to General and Aquatic Effects Monitoring	39

TABLE 1 – MONITORING GROUP	40

TABLE 2 – MONITORING PROGRAM	41

| Licence No. 2AM-MEL1631 Pursuant to the Nunavut Waters and Nunavut Surface Rights Tribunal Act and the Agreement Between the Inuit of the Nunavut Settlement Area and Her Majesty the Queen in Right of Canada, the Nunavut Water Board, hereinafter referred to as the Board, hereby grants to |  |
| --- | --- |
| A | GNICO-EAGLE MINES LIMITED |
| (Licensee) |  |
| 145, KING STREET EAST, SUITE 400, TORONTO, ONTARIO M5C 2Y7 |  |
| (Mailing Address) |  |
| hereinafter called the Licensee, the right to alter, divert or otherwise use Water or deposit Waste for a period subject to restrictions and conditions contained within this Licence: |  |
| Licence Number/Type: | 2AM-MEL1631 / Type “A” |
| Water Management Area: | WILSON WATERSHED (13) |
| Location: | MELIADINE GOLD PROJECT KIVALLIQ REGION, NUNAVUT |
| Purpose: | WATER USE AND DEPOSIT OF WASTE |
| Description: | MINING UNDERTAKING |
| Quantity of Water not to be Exceeded: | 62,000 CUBIC METRES ANNUALLY FOR CONSTRUCTION, 741,706 CUBIC METRES ANNUALLY FOR OPERATION, AND 4,000,000 CUBIC METRES ANNUALLY FOR CLOSURE (AS PER PART E) |
| Date Licence Issuance: | [Projected May 2021] |
| Expiry of Licence: | MARCH 31, 2031 |

This Licence issued (TBD) and recorded at Gjoa Haven, Nunavut includes and is subject to the annexed conditions.

| Lootie Toomasie Nunavut Water Board Hearing Chair | APPROVED BY: | Minister of Northern Affairs |
| --- | --- | --- |
|  | DATE LICENCE APPROVED: |  |

# PART A:	SCOPE, DEFINITIONS AND ENFORCEMENT

- SCOPE

  - This Licence authorizes Agnico-Eagle Mines Limited (“AEM” or “Licensee”) to use Waters and deposit of Waste in support of a Mining Undertaking classified as per Schedule 1 of the Regulations, at the Meliadine Gold Project (Project) as outlined in the Type “A” Water Licence Application (Application) submitted to the Nunavut Water Board (NWB or Board) on May 15, 2015 and the Type “A” Water Licence Amendment Application submitted to the NWB on August 2020, as reviewed throughout the regulatory process.

The Licensee may conduct mining, milling and associated activities at the Project in the Kivalliq Region of Nunavut, located at the following general geographic coordinates:

| Project Extents | Latitude | Longitude |
| --- | --- | --- |
| Mine Site | 63° 2' 53.091" N | 92° 16' 16.651" W |
|  | 63° 2' 50.722" N | 92° 9' 10.809" W |
|  | 63° 1' 1.463" N | 92° 9' 13.978" W |
|  | 63° 1' 3.829" N | 92° 16' 19.377" W |
| All-weather Access Road | 63° 1' 19.309" N | 92° 11' 26.684" W |
|  | 63° 1' 16.230" N | 92° 3' 10.432" W |
|  | 62° 47' 58.542" N | 92° 3' 36.080" W |
|  | 62° 48' 1.592" N | 92° 11' 48.601" W |
| Itivia, Rankin Inlet Area | 62° 48' 9.519" N | 92° 6' 4.112" W |
|  | 62° 48' 9.283" N | 92° 5' 27.421" W |
|  | 62° 47' 52.933" N | 92° 5' 27.925" W |
|  | 62° 47' 53.169" N | 92° 6' 4.610" W |
| Camp | 63° 2' 24.180" N | 92° 13' 44.288" W |

and including, in general, as follows:

    - Use of Water from Meliadine Lake for mining and milling, associated activities and domestic purposes;

    - Withdrawal and use of Water from Meliadine Lake for re-flooding of Tiriganiaq 1 and Tiriganiaq 2 open pits following pit development;

    - Dewatering of Lakes A54, H17, H19 and H20, and draining of ponds;

    - Quarrying of materials from specified locations;

    - Development and Operation of the site facilities;

    - Construction of access and site roads, Water crossings, industrial pad, and laydown areas;

- Construction and Operation of a potable water treatment plant and associated causeway and intake;

- Construction and Operation of a Sewage Treatment Plant (STP);

- Construction and Operation of a Landfill, Landfarm, and Incinerator;

- Construction and Operation of a water collection ponds, retention dikes, retention berms, jetties, dams, pump systems, pipeline, and channels;

- Construction and Operation of a contact Water Treatment Plant (WTP);

- Development and Operation of an underground Total Suspended Solids (TSS) removal plant;

- Construction and Operation of fuel tanks, dispensing storage facilities and associated secondary containment areas for the bulk storage of fuel at the Mine Site and at the Itivia Site Fuel Storage and Containment Facilities;

- Construction and Operation of the Rankin Inlet By-pass Road;

- Operation of the All-weather Access Road and associated infrastructure;

- Extraction of overburden, waste rock and ore from the Tiriganiaq gold deposit via two open pits and one underground mine;

- Construction and Operation of a temporary overburden stockpile;

- Construction and Operation of ore stockpiles;

- Construction and Operation of Waste Rock Storage Facilities;

- Construction and Operation of a Tailings Storage Facility (TSF);

- Processing of ore using a conventional gold-milling circuit;

- Development and Operation of an Emulsion Plant;

- Construction of berms required for the Operation of the Tailings Storage Facility (TSF);

- Deposition of dry stack tailings into the Tailings Storage Facility (TSF);

- Disposal of cyanide leach residue within the TSF;

- Disposal of Waste Rock and Overburden within three Waste Rock Storage Facilities (WRSF);

- Use of Waste Rock for Construction as approved by the Board in accordance with conditions of Part F;

- Management and disposal of wastes associated with the Sewage Treatment Plant, water collection ponds, Landfill, Landfarm, Incinerator and other wastes as described in the application;

  - Handling and storage of petroleum products and hazardous materials including explosives, cyanide and other reagents;

  - Diversion of site runoff Water to Water management facilities;

  - Controlled and regulated discharge of Effluent from the Water Treatment Plant, Control Pond No. 1 (CP1) to Meliadine Lake through an effluent diffuser; and

  - Progressive Reclamation and Abandonment planning of on-site facilities and infrastructure.

  - This Licence is issued subject to conditions contained herein with respect to the use  of Waters and the deposit of Waste of any type in any Waters or in any place under any conditions where such Waste or any other Waste that results from the deposits of such Waste may enter any Waters. Whenever new regulations are made under the Act or existing Regulations or Act are amended by the Governor in Council under the Act, or other statutes imposing more stringent conditions relating to the quantity, type or manner under which any such Waste may be so deposited are brought into force, this Licence shall be deemed to be  subject to such requirements.

  - Compliance with the terms and conditions of this Licence does not absolve the Licensee from responsibility for compliance with all applicable legislation, guidelines and directives.

# DEFINITIONS

  - The Licensee shall refer to Schedule A for definitions of terms used in this Licence.

# ENFORCEMENT

  - Failure to comply with this Licence may be a violation of the Act, subjecting the Licensee to the enforcement measures and the penalties provided for in the Act.

  - All inspection and enforcement services regarding this Licence will be provided by Inspectors appointed under the Act.

  - For the purpose of enforcing this Licence and with respect to the use of Waters and deposit of Waste by the Licensee, Inspectors appointed under the Act, hold all powers, privileges and protections that are conferred upon them by the Act or by  other applicable laws.

# PART B:	GENERAL CONDITIONS

- The amount of Water use fees shall be determined and payment of those fees shall be made by Licensee in accordance with section 12 of the Regulations.

- The Licensee shall file an Annual Report with the Board no later than March 31st in the

year following the calendar year being reported. The Annual Report shall be developed in accordance with Schedule B.

- The Licensee shall retain and have a copy of this Licence available at the site of operations at all times.

- Any communication with respect to this Licence shall be made in writing to the attention  of:

Manager of Licensing, Nunavut Water Board

P. O. Box 119

Gjoa Haven, NU X0B 1J0 Telephone:	(867) 360-6338

Fax:	(867) 360-6369

Email:	licensing@nwb-oen.ca

- Any notice made to an Inspector shall be made in writing to the attention of:

Manager of Field Operations

Nunavut District, Nunavut Region

P.O. Box 100

Iqaluit, NU X0A 0H0 Telephone:	(867) 975-4295

Fax:	(867) 979-6445

- The Licensee shall submit one (1) electronic copy of all reports, studies, and plans to the Board unless otherwise requested by the Board. Reports or studies submitted to the Board by the Licensee shall include an executive summary in English, Inuktitut, and French.

- This Licence is assignable as provided in Section 44 of the Act.

- The Licensee shall ensure that any document(s) or correspondence submitted by the Licensee to the Board is received and acknowledged by the Manager of Licensing or delegate.

- The Licensee shall notify the Board of any changes in Project phases associated with this Project at least thirty (30) days prior to any such change.

- The Licensee shall, for all Plans submitted under this Licence, include a proposed timetable for implementation. Plans submitted cannot be undertaken without subsequent written Board approval and direction, subject to the following exception. Plans submitted may be undertaken without subsequent written Board approval and direction after a 45 day period has elapsed following submissions by the Proponent with no action on the part of the Board. The Board may alter or modify a Plan if  necessary to  achieve the legislative objectives and will notify the Licensee in writing of acceptance, rejection or alteration of the Plan.

- Unless otherwise directed by the Board in writing, in the event that a Plan is not found acceptable to the Board, the Licensee shall provide a revised version to the Board for review within thirty (30) days of notification by the Board.

Page 4

- The Licensee shall, for all Plans submitted under this Licence, implement the Plan as approved by the Board in writing. The Board has approved (or accepted) the following Plans for implementation under the relevant sections in the Licence. Any changes to the plans deemed significant shall be  reviewed by the Board to determine the process for the Board’s review and approval of the amendment to the plan(s). Reflecting the scale and scope of the future changes of an approved plan, the Board may subsequently process the changes as solely an amendment to the plan, as a Modification under Part G of the Licence, or as an Amendment to the Licence. :

  - Ammonia Management Plan, Version 2, April 2020;

  - Aquatic Effects Monitoring Program (AEMP), June 2016;

  - Borrow Pits and Quarries Management Plan, Version 6, March 2018;

  - Bulk Storage Facility: Environmental Performance Monitoring Plan, Version 1, August 2019

  - Dust Management Plan, Version 6, June 2020

  - Environmental Management and Protection Plan (EMPP), Version 9, March 2019;

  - Explosives Management Plan, Version 6, March 2020;

  - Freshet Action Plan, Version 6, March 2020;

  - Groundwater Management Plan, Version 5, April 2020;

  - Hazardous Materials Management Plan, Version5, March 2018;

  - Incineration Management Plan, Version 6, February 2019;

  - Interim Closure and Reclamation Plan, December 2019;

  - Landfarm Management Plan, Version 3, March 2019;

  - Landfill and Waste Management Plan, Version7, March 2019;

  - Mine Plan (updated on a yearly basis);

  - Mine Waste Management Plan, Version 6, April 2020;

  - Ore Storage Management Plan, Version 2, March 2020;

  - Quality Assurance and Quality Control, Version 3, March 2019;

  - Risk Management and Emergency Response Plan, Version 4, April 2015;

  - Roads Management Plan, Version 8, December 2019;

  - Sediment and Erosion Management Plan, Version 2, March 2020;

  - Spill Contingency Plan, Version 10, December 2019;

  - Water Management Plan; Version 9, March 2020; and

  - Water Quality and Flow Monitoring Plan, Version 2, March 2020.

- The Licensee shall update for submission to the Board for review, within sixty (60) days of issuance of this Licence, the following management plans. The updates are to take into account commitments made with respect to submissions received during the technical review of the Application, as well as final submissions and issues raised during the Public Hearing process, where applicable.

- Every Plan to be carried out pursuant to the terms and conditions of this Licence shall become a part of this Licence, and any additional terms and conditions imposed upon approval of a Plan by the Board become part of this Licence. All terms and conditions of the Licence should be contemplated in the development of a Plan where appropriate.

- The Licensee shall review the Plans or Manuals referred to in this Licence as required by changes in operation and/or technology and modify the Plans or Manuals accordingly. Revisions to the Plans or Manuals are to be submitted in the form of an Addendum to be included with the Annual Report required by Part B, Item 2, complete with a revisions list detailing where significant content changes are made, and should incorporate design changes and adaptive engineering required and implemented during Construction and on the basis of actual site

conditions and monitoring results over the life of the Project, complete with a revisions list detailing where significant content changes are made.

- The Licensee shall post signs in the appropriate areas to inform the public of the location of the Water Supply Facilities and the Waste Disposal Facilities. All signs  must  be  in English and Inuktitut and shall be located and maintained to the satisfaction of an Inspector.

- The expiry or cancellation of this Licence does not relieve the Licensee from any  obligation imposed by the Licence, or any other regulatory requirement.

- The Schedules attached to this Licence provide details regarding the requirements associated with specific items in the main body of the Licence and are included in the Schedule to provide greater clarity and as an aid to interpretation for the Licensee. If the Board subsequently determines that an item in any of the Schedules requires revision in order to better reflect the intent and objectives of the Licence, the Board may at its discretion, and upon consulting and providing written notice to the Licensee and interested parties, revise the Schedule accordingly. Unless the Board directs otherwise, such revision may not necessarily be considered as an “Amendment” to the Licence.

- The Licensee is encouraged to adopt an Adaptive Management approach to the management of uncertainty regarding potential for effects associated with the Undertaking, including identifying mitigation, monitoring or management actions to be taken when specified thresholds and triggers identified in an Adaptive Management Plan are exceeded.

- Prior to the Licensee undertaking the mitigation, monitoring or management actions specified in an Adaptive Management Plan, the Licensee shall ensure that, reflecting the scale and scope of the actions proposed, all applicable regulatory requirements have been met, including, without limitation, applicable land use planning and impact assessment requirements under the Nunavut Agreement and the Nunavut Planning and Project Assessment Act, and completion of any Modification or Amendment processes required under the Act, the Regulations and/or this Licence.

- Unless otherwise stated, references in the Licence to any specific legislation, policy, guideline or other regulatory requirement are deemed to refer to the regulatory requirement as may be amended or as may be expressly replaced by successor legislation, policy, guidelines or other regulatory requirements after the Licence is approved by the Minister.

# PART C:	CONDITIONS APPLYING TO SECURITY

- The Licensee shall, within thirty (30) days following the approval of this Licence by the Minister, furnish and maintain security with the Minister in the amount of $. As set out in the Meliadine Security Management Agreement, , the amount  secured  under this  Part  constitutes  50% of the total  global  security amount    of

$that is required to reclaim the Undertaking and reflects that the other 50% of the global security amount will be held outside the Licence by the Kivalliq Inuit Association, in accordance with the terms and conditions of the Meliadine Security Management Agreement.

- The security held under Part C, Item 1 shall be in the form, of the nature, and subject to such terms and conditions, as prescribed by the Act and Regulations.

- The Licensee shall, within ten (10) days after furnishing security with the Minister, provide evidence to NWB and Kivalliq Inuit Association, that the security has been received by the Minister, indicating the amount, form, nature and conditions of the security.

- The Licensee shall, within ten (10) days after furnishing security with the Kivalliq Inuit Association, provide evidence to NWB and to the Minister, that it has been received by the Kivalliq Inuit Association, indicating the amount, form, nature and conditions of the security.

- The Licensee shall provide the Board with at least ninety (90) days written notice prior to any party’s termination of the Meliadine Security Management Agreement, or any material change to the Meliadine Security Management Agreement that may affect the amount of security  held under Part C, Item 1.

- The Licensee shall provide the Board with at least least ninety (90) days written notice prior to any material changes to the Undertaking or the risk of environmental damage associated with the Undertaking that could result in a material change to the reclamation liability associated with the Undertaking (including, but not limited to, updates to the reclamation cost estimate arising from unexpected changes or modifications of the works and activities associated with the Undertaking), a release, in whole or in part, of reclamation security held under this Part by the Minister pursuant to Part C, Item 11 and Section 76(5) of the Act.

- The Licensee shall, within six (6) months following commencement of Commercial Operation and at the time the Licensee files the Final Reclamation and Closure Plan as required under the Licence, submit to the Board for review in writing an updated reclamation cost estimate, using the RECLAIM Reclamation Cost Estimating Model (Version 7.0 or the most current version at the time the updated reclamation cost estimate is submitted to the Board).

- Upon the Board receiving notice under Part C, Items 5 or 6, or upon receiving an updated reclamation cost estimate as required under Part C, Item 7, the Board, may on its own initiative, or upon application by the Licensee, the Minister and/or Kivalliq Inuit Association, conduct a periodic review of the outstanding reclamation liability associated with the Undertaking and may, as the Board considers appropriate, amend the amount of security held under Part C, Item 1. Any submission requesting an amendment to the security provisions of the Licence shall include supporting evidence to justify the amendment and will be processed by the Board as an amendment to the terms and conditions of the Licence.

- In addition to the process for amending security under Part C, Item 8, the Licensee may, at any time, submit an application to the Board for a change to the amount of security outlined in Part C, Item 1. The submission shall include supporting evidence to justify the amendment. The Licensee’s request to amend security will be processed by the Board as an amendment to the terms and conditions of the Licence. For greater clarity, such amendments may not require a Public Hearing.

- If the Board determines it to be necessary, or upon the request by Licensee, the Minister and/or the Kivalliq Inuit Association, the Board may issue further directions under this Part with respect to the process for the Board’s conduct of periodic reviews of security and associated amendments to the amount of security to be furnished and maintained under the Licence.

- The Licensee shall maintain the security deposit referred to in Part C, Item 1 until such time as the Minister is satisfied that the Licensee has complied with all provisions of the approved Abandonment and Restoration Plan. This clause shall survive the expiry of the Licence or renewals thereof and until full and final reclamation has been completed to the satisfaction of the Minister.

# PART D:	CONDITIONS APPLYING TO CONSTRUCTION

- The Licensee shall submit to the Board for approval, at least thirty (30) days prior to Construction, final design and Construction drawings accompanied, with a detailed report in Part D, Item 2 and stamped and signed by an Engineer, for the following:

  - Engineered Water works, including: Water Intake and causeway water control structures (dikes, berms, jetties, channels) and Water crossings (culverts, bridges);

  - Engineered Waste disposal facilities including: Contact Water Treatment Plant, Sewage Treatment Plant, Discharge Diffuser, Waste Rock Storage Facilities, Overburden stockpiles, Tailings Storage Facility (including thermal installation and monitoring),

Landfill, Landfarm, and Incinerator; and

  - Engineered Fuel Storage Containment Facilities (Meliadine Site and Itivia Site).

- The detailed report(s) referred to in Part D, Item 1 shall include:

  - Design rationale, requirements, criteria, parameters, standards analysis, methods, assumptions and limitations;

  - Site specific data and analysis  to support the design and management decisions;

  - Geochemical analysis of Waste Rock and fill, demonstrating their Acid Rock Drainage and Metal Leaching characteristics;

  - Construction methods and procedures regarding how infrastructure will be put in place, including quality assurance and quality control measures and equipment to be used;

  - Technical specifications for sedimentation, erosion control and bank stabilization measures, including proposed materials, location and extent, place methods and quantities required; and

  - Timetable for submission, including date of Construction and proposed date of commissioning of infrastructure

- The Licensee shall submit to the Board for review, within ninety (90) days of completion  of each facility designed to contain, withhold, divert or retain Waters or wastes, a Construction Summary Report prepared by a qualified Engineer that includes as-built plans and drawings, documentation of field decisions that deviate from original plans and any data used to support these decisions in accordance with Schedule D, Item 1.

- The Licensee shall provide a brief summary of the Construction Summary Report required by Part D, Item 3, within the Annual Report required by Part B, Item 2.

- The Licensee shall construct and operate the Fuel Storage and Containment Facilities to meet, at a minimum, all applicable legislation and industry standards that include the following:

  - Environmental Code of Practice for Aboveground and Underground Storage Tank Systems Containing Petroleum and Allied Petroleum Products, 2003; CCME PN1326 (Updated in 2013) or most recent; and

  - National Fire Code, 2010 or most recent.

- The Licensee shall cease works on water crossings/bridges, should the results of downstream monitoring under Part I, Item 11 exceed the upstream monitoring results for Total Suspended Solids concentration (mg/L) by twenty percent (20%).

- The Licensee shall use Waste Rock and fill material for construction, including the construction of any infrastructure, only from approved sources that have been demonstrated by appropriate geochemical analyses to not produce Acid Rock Drainage and to be Non-Metal Leaching, and free of contaminants.

- The Licensee shall monitor for signs of erosion and implement and maintain sediment and erosion control measures prior to and during the Construction and Operation where necessary to prevent entry of sediment into Water.

- The Licensee shall conduct daily visual inspections for Construction activity during spring freshet and during and after remarkable rainfall events, with sampling of runoff/Seepage where turbidity is evident.

- The Licensee shall construct and maintain all containment and runoff control structures to prevent escape of Wastes to surface Waters.

- The Licensee shall direct contact runoff and Seepage to the Collection Ponds for storage and transfer to the Control Pond No.1 (CP1).

- All Waters from dewatering activities at Monitoring Program Stations MEL-D-1 through MEL-D-TBD shall be directed to Meliadine Lake and shall not exceed the following Effluent quality limits:

| Parameter | Maximum Average Concentration | Maximum Concentration of Any Grab Sample |
| --- | --- | --- |
| Total Suspended Solids (TSS) (mg/L) | 15.0 | 30.0 |
| pH | 6.0 to 9.5 | 6.0 to 9.5 |

- All Waters, exceeding the Effluent quality limits under Part D, Item 12, shall be released to CP1.

- The Licensee shall implement the Borrow Pits and Quarries Management Plan, and the Roads Management Plan, as approved by the Board under Part B, Item 12.

- The Licensee shall designate an area for the deposition of excavated and stockpiled materials, with respect to access roads, laydown area, pad construction or other earthworks, at a distance of at least thirty-one (31) metres from the ordinary High Water Mark, in order to prevent the deposition of debris or sediment into or onto any Water body.

- The Licensee shall maintain a minimum of thirty-one (31) metres undisturbed buffer zone between the periphery of quarry sites and the ordinary High Water Mark of any Water body unless otherwise approved by the Board in writing.

- The Licensee shall not excavate and/or remove material from the quarry beyond a depth of one (1) metre above the ordinary High Water Mark or above the groundwater table, to prevent the potential contamination of surface and groundwater. The quarrying shall be in accordance with all applicable legislation and industry standards including the Northern Land Use Guidelines, Pits and Quarries (INAC, 2009, or as revised).

- All surface runoff and/or discharge from drainage management systems, at the Monitoring Program Stations MEL-SR-1 to MEL-SR-TBD referred to in Part I, Item 11, during the Construction/Operation of any facilities and infrastructure associated with this project, including laydown areas and All-weather Access Road, where flow may directly or indirectly enter a Water body, shall not exceed the following Effluent quality limits:

| Parameter | Maximum Average Concentration | Maximum Concentration of Any Grab Sample |
| --- | --- | --- |
| Total Suspended Solids (TSS) (mg/L) | 50.0 | 100.0 |
| Oil and Grease | No Visible Sheen | No Visible Sheen |
| pH | Between 6.0 and 9.5 | Between 6.0 and 9.5 |

- The Licensee shall supervise and field check through an appropriately qualified Engineer, all Construction of engineered structures in such a manner that the project specification can be enforced and, where required, the quality control measures are followed. The Licensee shall maintain and make available at the request of the Board and/or an Inspector, all Construction records of all engineered structures.

- The Licensee shall undertake appropriate corrective measures to mitigate impacts on surface drainage resulting from the Licensee’s Operations.

- The Licensee shall locate stream crossings to minimize approach grades. Approaches shall be stabilized during Construction and upon completion of the project, to control runoff, erosion and subsequent siltation to any Water body.

- The Licensee shall limit any in-stream activity to low Water periods. In-stream activity is prohibited during fish migration.

- The Licensee shall not cut any stream bank or remove any material from below  the ordinary High Water Mark of any Water body.

- The Licensee shall, for the purposes of culvert and bridge construction, ensure that all activities  remain  outside  of  the  natural  channel  width  by  the  placement  of abutments,

footings or armouring above the ordinary High Water Mark so that there is no restriction to the natural channel processes.

- Machinery is not permitted to travel up the stream bed and fording of any Water body is to be kept to a minimum. Machinery and equipment should be well cleaned and free of oil  and grease and other pollutants and maintained free of fluid leaks.

- The Licensee shall ensure that pollutants from machinery fording the crossings do not enter Water.

- The Licensee shall locate equipment storage areas on gravel, sand or other durable land, at a distance of at least thirty one (31) metres above the ordinary High Water Mark of any Water body in order to minimize impacts on Surface Drainage and water quality.

- The Licensee shall not utilize any equipment or vehicles in the course of this undertaking unless the ground surface is in a state capable of supporting the equipment or vehicles without rutting or gouging. Overland travel of equipment or vehicles shall cease if rutting occurs.

- The Licensee shall not store material on the surface of frozen streams or lakes except what is for immediate use.

- The Licensee shall determine all monitoring locations based on operational and site  specific requirements, to be recorded and reported within the monthly and annual reports required under Part B, Schedule B and Part I.

# PART E:	CONDITIONS APPLYING TO WATER USE AND MANAGEMENT

- The Licensee shall obtain all fresh Water for domestic camp use, mining and milling and associated uses, from Meliadine Lake at Monitoring Program Station MEL-11 using the Fresh Water Intake, or as otherwise approved by the Board in writing. The total authorized volume of Waters for all purposes shall not exceed sixty two thousand (62,000) cubic meters per year during Construction and seven hundred and forty one thousand, seven hundred and six (741,706) cubic meters per year during Operations of the Project.

- The Licensee shall obtain all fresh Water from Meliadine Lake for domestic camp use and re-flooding of Tiriganiaq 1 and 2 Pits and associated uses, or as otherwise approved by the Board in writing. The total authorized volume of Waters for re-flooding of pits shall not exceed four million (4,000,000) cubic metres per year during closure of the Project.

- The Licensee shall obtain Water for use in dust suppression from ponded Water (against the AWAR), small ponds proximal to the road and/or Water obtained from the Meliadine River, or as otherwise approved by the Board in writing.

- Where the use of Water for drilling and/or dust suppression is of a sufficient volume that the source Water body may be drawn down, the Licensee shall request approval of the Board in writing

- The Licensee shall maximize to the greatest practical extent, the use of Reclaim Water  from contact Water management facilities, for use in the mill, drilling, and for dust suppression.

- The Licensee shall not use streams as a Water source unless authorized and approved by  the Board in writing.

- The Licensee shall equip all Water intake hoses with a screen of an appropriate mesh size  to ensure that fish are not entrained and shall withdraw Water at a rate such that fish do not become impinged on the screen.

- The Licensee shall not remove any material from below the ordinary High Water Mark of any Water body unless authorized by the Board in writing.

- The Licensee shall undertake appropriate corrective measures to prevent and/or mitigate impacts to surface Water resulting from the Licensee’s Operation.

- The Licensee shall implement sediment and erosion control measures prior to and maintain such measures during the undertaking to prevent entry of sediment into Water.

- The Licensee shall implement the Water Management Plan, as approved by the Board under Part B, Item 12. The Licensee shall update the Water Management Plan for submission to the Board for review, when significant content changes are required.

- The Licensee shall submit an updated Water Management Plan to the Board for approval in writing, at least ninety (90) days prior to starting of the next phase (Closure, Post-closure) of mine development to reflect all changes in operations and/or technology. The Plan shall include updated Water Balance and Water Quality Forecast, and an action  plan to be implemented, if predicted Water quality indicates that treatment is necessary.

- The Licensee shall review the Water Management Plan on an annual basis and provide a summary of changes, if any, in the annual report. A revised Water Management Plan shall be submitted to the Board for review, when significant content changes are required.

- An updated Water Balance and Water Quality Forecast will be provided at a minimum of every two (2) years following commencement of Operations. This update will focus on the regulated parameters listed in Part F, Item 3 that are within 10% of the annual maximum average concentration discharge limit.

- The Licensee shall, on an annual basis during Closure, compare the predicted Water quantity and quality within the pits, to the actual measured water quantity and quality. Should the difference between the predicted and measured values be 20% or greater, then the cause(s) of the difference(s) shall be identified and the implications of the differences shall be assessed and reported to the Board.

- The Licensee shall implement the Groundwater Management Plan, as approved by the Board under Part B, Item 12. The Licensee shall update the Groundwater Management Plan for submission to the Board for review, when significant content changes are required.

- The Licensee shall carry out Weekly inspections of all Water management  structures during periods of flow (rock drains, culverts, sedimentation and pollution control ponds  and associated diversion berms, reagent and storage facility sumps, and the sedimentation control berm at the overburden dump) and Monthly thereafter and the records be maintained for review upon request of an Inspector. More frequent inspections may be required at the request of an Inspector.

- The Licensee shall not breach dikes until the water quality in the pit re-flooded area has been shown to be less than or equal to the CCME Water Quality Guidelines for the Protection of Aquatic Life, appropriate site specific water quality objectives, or the pit lake predictions (Agnico Eagle 2014, FEIS, Table 7.4-22). If water quality parameters are above CCME Guidelines and/or FEIS predictions, a site specific risk assessment must be conducted in order to identify Site Specific Water Quality Objectives (SSWQO’s) for the site that that are protective of the aquatic environment. Where they are required, Site Specific Water Quality Objectives shall be incorporated in the approved Final Reclamation and Closure Plan.

- The Licensee shall implement measures to minimize the generation and deposition of dust and/or sediment into Water arising from road use.

- The Licensee shall provide at least thirty (30) days’ notice to the NWB and an Inspector prior to the start of the next phase (Closure, Post-closure) of mine development including a notice to change Water use for Operations and notice to increase Water use for re-flooding of pits during Closure .

# PART F:	CONDITIONS APPLYING TO WASTE DISPOSAL AND MANAGEMENT

- The Licensee shall direct all Sewage and Greywater to the Sewage Treatment Plant for treatment prior to releasing to CP1, or as otherwise approved by the Board in writing.

- The Licensee shall direct all Contact Water from the Collection Ponds to CP1, or as otherwise approved by the Board in writing.

- The Discharge of Effluent from the Final Discharge Point at Monitoring Program Station MEL-14 shall be directed to Meliadine Lake through the Meliadine Lake Outfall Diffuser and shall not exceed the following Effluent quality limits:

| Parameter | Maximum Average Concentration | Maximum Concentration of Any Grab Sample |
| --- | --- | --- |
| pH | 6.0 to 9.5(a) | 6.0 to 9.5(a) |
| TDS (mg/L) (measured) | 3,500 | 5,000 |
| TSS (mg/L) | 15(a) | 30(a) |
| Total (T)-P (mg/L) | 2.0 | 4.0 |
| T-Al (mg/L) | 2 | 3 |
| T-As (mg/L) | 0.3 | 0.6 |
| T-CN (mg/L) | 0.5 | 1.0 |
| T-Cu (mg/L) | 0.2 | 0.4 |
| NH4-N (mg/L) | 14 | 18 |
| T-Ni (mg/L) | 0.5(a) | 1.0(a) |
| T-Pb (mg/L) | 0.2(a) | 0.4(a) |
| T-Zn (mg/L) | 0.4 | 0.8 |
| Total Petroleum Hydrocarbons (TPH) (mg/L) | 5 | 5 |

Metal and Diamond Mining Effluent Regulations (SOR/2002-222) continue to apply and may include additional parameters or different concentrations than those described above at any point in time.

- The Discharge of Effluent from the Final Discharge Point at Monitoring Program Station MEL-14 shall be demonstrated to be non-Acutely Lethal

a.

- The Discharge of Effluent onto land from the Fuel Storage Facility at the Itivia Site, Monitoring Program Station MEL-25, shall not exceed the following Effluent  quality limits:

| Parameter | Maximum Average Concentration | Maximum Concentration of any Grab sample |
| --- | --- | --- |
| pH | 6.0 to 9.5 | 6.0 to 9.5 |
| Total Suspended Solids (mg/L) | 15 | 30 |
| Benzene (µg/L) | 370 | 370 |
| Toluene (µg/L) | 2 | 2 |
| Ethylbenzene (µg/L) | 90 | 90 |
| T-Pb (mg/L) | 0.1 | 0.1 |
| Oil and Grease (mg/L) | 5 and no visible sheen | 5 and no visible sheen |

- The Licensee shall, under Part F, Item 5, discharge Effluent to a location at a distance of at least thirty-one (31) metres above the ordinary High Water Mark of any Water body, in such a manner as to minimize surface erosion to where direct flow into a Water body is not possible and no additional impacts are created, or as otherwise approved by the Board in writing.

- All Effluent at Monitoring Station MEL-25, that exceeds the Effluent quality limits under Part F, Item 5, shall be transferred to CP1.

- The Licensee shall confirm compliance with Effluent quality limits referred to in Part F, Items 3 and 5 prior to Discharge.

- The Licensee shall submit to the Board for approval in writing an Operation and Maintenance Manual(s) for the Wastewater Treatment Plant(s), including the Sewage Treatment Plant, at least ninety (90) days prior to the construction/installation of facilities, prepared in accordance with the “Guidelines for the Preparation of an Operation and Maintenance Manual for Sewage and Solid Waste Disposal Facilities in the Northwest Territories; 1996”. The Manual(s) shall include sludge management and disposal information and contingency measures in the event of plant malfunction.

- The Licensee shall provide at least ten (10) day notice to the Inspector prior to any planned Discharge from any facility. The notice shall include the estimated volume proposed for Discharge and the receiving location.

- The Licensee shall locate areas designated for Waste disposal at a minimum distance of thirty-one (31) metres from the ordinary High Water Mark of any Water body such that the quality, quantity or flow of Water is not impaired, or as otherwise approved by the Board  in writing.

- The Licensee shall implement the Mine Waste Management Plan, as approved by the Board under Part B Item 12. The Licensee shall update the Mine Waste Management Plan for submission to the Board for review, when significant content changes are required.

- The Licensee is authorized to dispose of and contain all non-hazardous, solid Wastes at the Meliadine Non-Hazardous Waste Landfill in accordance with the Landfill Design and Waste Management Plan”, as approved by the Board under Part B, Item 12.

- The Licensee shall manage domestic waste and other items as per the Incineration Management Plan, as approved by the Board under Part B, Item 12, which may include an adaptive management approach for use of a composter or other options.

- The Licensee shall not open burn plastics, wood treated with preservatives, electric wire, Styrofoam, asbestos or painted wood to prevent the deposition of waste materials of incomplete combustion and/or leachate from contaminated ash residual, from impacting any surrounding waters, unless otherwise approved by the Board in writing.

- The Licensee shall remove from the Project site, all solid and liquid Hazardous Wastes generated through the course of the project’s activities, for disposal at an approved hazardous waste disposal facility in accordance with the Hazardous Materials Management Plan, as approved by the Board under Part B, Item 12 .

- The Licensee shall incorporate Seepage management at Quarries using best management practices including ditches, diversions, sumps and berms where necessary.

- The Licensee shall maintain records of all Waste backhauled and confirmation of proper disposal through the use of Waste manifest tracking systems and registration with the Government of Nunavut, Department of Environment. These records shall be made available to an Inspector upon request.

- The Licensee shall dispose/treat of all petroleum hydrocarbon contaminated soils in the Landfarm in accordance with the Landfarm Management Plan, as approved by the Board under Part B, Item 12.

- Licensee shall dispose of tailings and operate the Tailings Storage Facility in accordance with the Mine Waste Management Plan, as approved by the Board under Part B, Item 12, Guide to the Management of Tailings Facilities (Mining Association of Canada September 2011 or more recent) and with the engineering standards such that:

- Implement Adaptive Management strategies as required;

- Seepage from the TSF shall be collected in Collection Pond No.3 (CP3) and monitored for water quality;

- The Licensee shall carry out, at a minimum, weekly inspections during any period in which the site is occupied and Water is being actively managed, to identify and remediate where necessary, areas of concern including issues of Seepage, cracking, and ponding for all structures and other associated structures; and

- The Licensee shall consult the Geotechnical Engineer when significant issues associated with the TSF are observed and implement the Engineer’s recommendations as necessary;

- The solids fractions of all mill tailings (except for mill tailings that will be used for paste backfill in the underground) shall be deposited and permanently contained within the TSF;

- An annual Geotechnical inspection shall be carried out in accordance with Part I, Item 14;

- The Licensee shall perform more frequent inspections of the facilities at the request  of an Inspector;

- The Licensee shall maintain records of all inspections for the review of an Inspector upon request.

# PART G:	CONDITIONS APPLYING TO MODIFICATIONS

- The Licensee may, without written consent from the Board, carry out Modifications provided that such Modifications are consistent with the terms of this Licence and the following requirements are met:

  - The Licensee has notified the Board in writing of such proposed Modifications at  least sixty (60) days prior to beginning the Modifications;

  - Such Modifications do not place the Licensee in contravention of the Licence or the Act;

  - Such Modifications are consistent with the NPC Land Use Planning and NIRB Screening Determinations;

  - The Board has not, within sixty (60) days following notification of the proposed Modifications, informed the Licensee that review of the proposal will require more than sixty (60) days; and

  - The Board has not rejected the proposed Modifications.

- Modifications for which any of the conditions referred to in Part G, Item 1 have not been met can be carried out only with approval from the Board in writing.

- Applications for modifications shall contain:

  - A description of the facilities and/or works to be constructed;

  - The proposed location of the structure(s);

  - Identification of any potential impacts to the Receiving Environment;

  - A description of any monitoring required, including sampling locations, parameters measured, and frequencies of sampling;

  - Schedule for construction;

  - Drawings of Engineered Structures stamped by an Engineer; and

  - Proposed sediment and erosion control measures.

- The Licensee shall provide to the Board, within ninety (90) days of completion of the Modification, as-built plans and drawings of the Modifications referred to in this Part. These plans and drawings shall be stamped by an Engineer.

# PART H:

# CONDITIONS APPLYING TO EMERGENCY RESPONSE AND CONTINGENCY PLANNING

- The Licensee shall implement and maintain the Spill Contingency Plan,  the Hazardous Materials Management Plan, and the Risk Management and Emergency Response Plan, , as approved by the Board under Part B, Item 12. The Licensee shall comply with the Plan(s), and any changes deemed significant shall require the submission and subsequent approval of the Board in writing.

- The Licensee shall prevent any chemicals, petroleum products or unauthorized Wastes associated with the Project from entering Water.

- The Licensee shall provide secondary containment for fuel and chemical storage as  required by applicable standards and acceptable industry practice.

- The Licensee shall perform weekly inspections of Fuel Storage and Containment Facilities, for leaks and settlement and shall keep a written log of inspections to be made available to an Inspector upon request.  More frequent inspections may be requested by an Inspector.

- The Licensee shall, upon providing notification with respect to Care and Maintenance  under Part J, Item 2, shall submit to the Board an Addendum to the Emergency Response Plan and the Spill Contingency Plan, detailing the changes in operations, personnel, responsibilities, availability of equipment and access to the site for assistance.

- The Licensee shall keep a copy of the Emergency Response Plan and the Spill Contingency Plan at each site of operation.

- The Licensee shall conduct emergency maintenance and servicing on equipment, in designated areas, and shall implement measures to collect motor fluids and other Waste to prevent and contain spills.

- The Licensee shall, subject to Section 16 of the Regulations, report any unauthorized deposits or foreseeable unauthorized deposits of waste and/or discharges of Effluent, and:

  - Employ the Spill Contingency Plan;

  - Report the incident immediately via the 24-Hour NWT/NU Spill Reporting Line (867) 920- 8130 and to the Inspector at (867) 975-4295; and

  - For each spill occurrence, submit a detailed report to the Inspector, no later than thirty (30) days after initially reporting the event, which includes the amount and type of spilled product, the GPS location of the spill, and the measures taken to contain and clean up the spill site.

- The Licensee shall, in addition to Part H, Item 8, regardless of the quantity of release of a harmful substance, report to the 24 hour NWT/NU Spill Reporting Line if the release is near or into a Water body.

# PART I:

# CONDITIONS APPLYING TO GENERAL AND AQUATIC EFFECTS MONITORING

- The Licensee shall implement the Environmental Management and Protection Plan, , as approved by the Board under Part B, Item 12.

- The Licensee shall install and maintain flow meters or other such devices, or implement suitable methods required for the measuring of the Use of Water and Effluent discharge volumes, to be operated and maintained to the satisfaction of an Inspector.

- The Licensee shall undertake the Water Monitoring Program provided in the Tables 1 and  2 of Schedule I. The Licensee shall establish the locations and GPS coordinates for all monitoring stations in consultation with an Inspector.

- The Licensee shall install and maintain signs that identify monitoring stations. The signs shall be posted in English and Inuktitut.

- The Licensee shall conduct Acute Lethality Testing, at monitoring station MEL-14, in accordance with Part F, Item 4 and Schedule I.

- The Licensee shall measure and record the following on a Monthly basis in cubic metres or as otherwise stated:

  - The volume of fresh Water obtained from Meliadine Lake at Monitoring Program Station MEL-11;

  - The volume of fresh Water transferred to the Meliadine Lake during lakes’ dewatering activities;

  - The volume of fresh Water obtained along the road and Meliadine River for dust suppression activities;

  - The volume of Effluent discharged from Final Discharge Point at Monitoring  Program Station MEL-14;

  - The volume of reclaim Water obtained from the CP1;

  - The volume of Effluent discharged onto tundra at Monitoring Program Station MEL- 25 or transferred to CP1 from the Itivia Site Fuel Storage and Containment Facility;

  - The volume of Effluent and Fresh Water transferred to the pits during pits’ flooding;

  - The volume of Sewage sludge removed from the Sewage Treatment Plant and the locations or methods of Sewage sludge disposal;

  - Quantity of waste placed within the Landfill and Landfarm;

  - Tonnes of ore stockpiled and ore processed through the mill;

  - Tonnes of waste rocks placed within the Waste Rock Storage Facilities; and

  - The daily tonnes of dry combined tailings placed within the Tailings Storage Facility.

- The Licensee shall submit to the Board, within thirty (30) days following the month being reported, a Monthly Monitoring Report.  The Report shall include:

  - All data and information required by this Part and generated by the Monitoring Program in the Tables of Schedule I;

  - An assessment of data to identify areas of non-compliance with regulated discharge parameters referred to in Parts D and F.

- The Licensee shall complete water quality testing immediately upstream and downstream  of water crossings, any significant Water seeps in contact with the roads and any significant Water seeps/runoff originating from borrow pits and quarries, during blasting activities, periods of flow and following significant precipitation events at Monitoring Program Stations MEL-SR-1 through MEL-SR-TBD, prior to Construction, on a weekly  basis during Construction and on a monthly basis upon completion of construction,  in accordance with the Part D, Item 18 and Schedule I of the Licence.

- The Licensee shall undertake the Waste Rock Storage Facilities’ and Tailings Storage Facility’s Thermal Monitoring Program detailed in the Environmental Management and Protection Plan, and Mine Waste Management Plan, as approved by the Board under Part B, Item 12.

- The Licensee shall undertake a geotechnical inspection, to be carried out annually by a Geotechnical Engineer, between the months of July and September.  The inspection shall  be conducted in accordance with the Canadian Dam Safety Guidelines where applicable and take into account all major earthworks, included within Schedule I.

- The Licensee shall submit to the Board as part of the Annual Report required by Part B, Item 2, a Geotechnical Engineer’s Inspection Report. The Report shall include a cover  letter from the Licensee outlining an implementation plan addressing each of the Geotechnical Engineer’s recommendations.

- The Licensee shall obtain a digital photographic record of all the watercourse crossings before, during, and after construction has been completed.

- The Licensee shall maintain a Quality Assurance / Quality Control Plan, accepted by the Board that includes a cover letter from the accredited laboratory confirming approval of the Plan for analyses to be performed under this Licence. The QA/QC Plan shall be prepared and updated as needed in accordance with and in consultation with the accredited laboratory conducting the analyses. This Plan shall be developed in accordance with  current Standard Methods and the 1996 Quality Assurance (QA) and Quality Control (QC) Guidelines for Use by Class “A” (INAC).

- The Licensee shall annually review the approved QA/QC Plan and modify the Plan as necessary.  Proposed changes shall be submitted to an Accredited Laboratory for approval.

- All analyses shall be conducted as described in the most recent edition of “Standard Methods for the Examination of Water and Wastewater” or by other such methods approved by an Analyst.

- All compliance analyses shall be performed in an accredited laboratory according to ISO/IEC Standard 17025.  The accreditation shall be current and in good standing.

- The NWB can modify the Monitoring Program as set out in Schedule I without a public hearing. Requests for changes to the Program should be forwarded to the NWB in writing, and should include the justification for the change.

- Additional monitoring may be imposed by the Board or by the Inspector.

# PART J:

# CONDITIONS APPLYING TO ABANDONMENT, RECLAMATION AND CLOSURE

- The Board has approved the document entitled the Interim Closure and Reclamation Plan, under Part B, Item 12 in accordance with the Mine Site Reclamation Guidelines for the Northwest Territories, 2007 and consistent with the INAC Mine Site Reclamation Policy for Nunavut, 2002.  The Plan shall include the following:

  - Detailed description, including maps and other visual representations, of the pre- Construction conditions for each site, accompanied by a detailed description of the proposed final landscape, with emphasis on the reclamation of surface drainage over the restored area;

  - A description of how progressive reclamation will be employed and monitored throughout the life of the mine, plus reclamation scheduling and coordination of activities with the overall sequence of the project; details of reclamation scheduling and procedures for coordinating reclamation activities within the overall mining sequence and materials balance;

  - Implications of water quality model re-calibration results on discharge strategy and any adaptive management measures that may be required;

  - An evaluation of closure and reclamation measures for each mine component, including the goals, objectives, closure criteria and the rationale for selection of the preferred measures;

  - A comprehensive assessment of materials suitability, including geochemical and physical characterization, and schedule of availability for reclamation needs, with attention to cover materials, including maps where appropriate, showing sources and stockpile locations of all reclamation construction materials and any Water related mitigation required during implementation;

  - An assessment and description of any required post-closure treatment for drainage Water that is not acceptable for discharge from any of the reclaimed mine components;

  - Contingency measures for all reclamation components including action thresholds  that are linked to the monitoring programs;

  - Monitoring programs to assess reclamation performance and environmental conditions including monitoring locations for surface Water and groundwater, parameters, schedules and overall timeframes;

  - QA/QC procedures for managing all waste disposal areas;

  - A list of non-salvageable materials and disposal locations;

  - Rock storage facility closure design plans and sections including the types of material placed and volumes;

  - Underground mine plans and sections, including the areas of backfill, the type of material placed and volumes;

  - Protocol for the disposal of any contaminated soil at closure;

  - An assessment of the long-term physical stability of project components;

  - A detailed criteria for the final breaching of the Dam(s);

  - A revised closure and reclamation cost estimate; and

  - A detailed implementation schedule for completion of reclamation work.

- The Licensee shall notify the Board in writing, at least sixty (60) days prior to, or as soon  as practically possible, the intent to enter into a Care and Maintenance Phase.

- The Licensee shall provide the Board for review, within thirty (30) days of the Licensee providing notice of intent to enter into Care and Maintenance under Part J, Item 2, a Care and Maintenance Plan that details the Licensee’s plans for maintaining compliance with the Terms and Conditions of the Licence.

- The Licensee shall provide the Board for approval in writing, within ninety (90) days of the Licensee providing a notice of intent to enter into Care and Maintenance under Part J, Item 2, all operational revised Plans to reflect the Care and Maintenance status.

- The Licensee shall submit to the Board for approval at least twelve (12) months prior to the expected end of planned mining, a Final Closure and Reclamation Plan. The Final Plan shall incorporate revisions, which reflect the pending closed status of the mine, and  include:

  - Soil Quality Remediation Objectives along with CCME Guidelines and the Government of Nunavut Environmental Guideline for Site Remediation;

  - Environmental Site Assessment plans in accordance Canadian Standards Association (CSA) criteria; and

  - An evaluation of the Human Health and Ecological Risk associated with closure options.

- The Licensee shall complete all reclamation work in accordance with the Plan(s) referred to in this Part as and when approved by the Board in writing.

- The Licensee shall review the Plans referred to in this Part as required by changes in operation  and/or technology and  modify the Plans  accordingly.   Revisions  to  the   Plans

should incorporate design changes and adaptive engineering required and implemented during Construction and on the basis of actual site conditions and monitoring results over the life of the Project.

- The Licensee shall implement progressive reclamation, including progressive covering of the tailings, and re-vegetation if practically possible.

# SCHEDULES

Schedule A:	Scope, Definition, and Enforcement Schedule B:	General Conditions

Schedule C:	No Schedule for Security

Schedule D:	Conditions Applying to Construction Schedule E:	No Schedule for Water Use and Management

Schedule F	No Schedule for Waste Disposal and Management Schedule G:	No Schedule for Modifications

Schedule H:	No Schedule for Emergency Response and Contingency Planning Schedule I:	Conditions Applying to General and Aquatic Effects Monitoring Schedule J:	No Schedule for Abandonment, Reclamation and Closure

# Schedule A:	Scope, Definitions, and Enforcement

In this Licence: 2AM-MEL1631

“Abandonment” means the permanent dismantlement of a facility so it is permanently incapable of its intended use.  This includes the removal of associated equipment and structures;

“Act” means the Nunavut Waters and Nunavut Surface Rights Tribunal Act;

“Acid Rock Drainage (ARD)” means the production of acidic leachate, Seepage or drainage from underground workings, open pits, ore piles, waste rock, construction rock that can lead to the release of metals to groundwater or surface Water during the life of the Project and after closure;

“Acutely Lethal” means effluent as defined in the Metal and Diamond Mining Effluent Regulations (SOR/2002-222 dated June 6, 2002 and  as may be further amended from time to time).

“Adaptive Management” means a management approach that describes a way of managing risks associated with uncertainty and provides a flexible framework for mitigation, monitoring and management measures to be implemented and actions to be taken when specified thresholds are exceeded. Measures may include special studies, operational changes, revised or new water and waste management systems, structures and/or facilities or implementing mitigation activities to prevent, stabilize or reverse a change in environmental conditions or otherwise protect the Receiving Environment;

“Addendum” means the supplemental text that is added to a full plan or report, usually included at the end of the document and is not intended to require a full resubmission of the revised report.  It may also be considered as an appendix or supplement;

“All Weather Access Road” means an All Weather Access Road (AWAR) and associated water crossings between the Hamlet of Rankin Inlet and the Meliadine Gold mine site as described in the Roads Management Plan;

“Amendment” ; means a change to any terms and conditions of this Licence through application to the NWB, requiring a change, addition, or deletion of specific terms and conditions of the Licence not considered as a Modification;

“Analyst” means an Analyst designated by the Minister under section 85 (1) of the Act;

“Annually” means, in the context of monitoring frequency, one sampling event occurring every 365 days with a minimum of 200 days between sampling events;

“Appurtenant Undertaking” means an undertaking in relation to which a use of Water or a deposit of waste is permitted by a licence issued by the Board;

“Aquatic Effects Monitoring Program (AEMP)” means a monitoring program designed to determine the short and long-term effects in the aquatic environment resulting from the Project, to evaluate the accuracy of impact predictions, to assess the effectiveness of planned impact mitigation measures and to identify additional impact mitigation measures to avert or reduce environmental effects;

“Board” means the Nunavut Water Board established under Article 13 of the Nunavut Agreement and under section 14 of the Act;

“Borrow Pits” means sites used for the purpose of extracting materials, such as gravel or sand, for the construction of site infrastructure and facilities;

“By-pass Road” means access, service, and haul roads and associated water crossings  as described in the Roads Management Plan;

“Canadian Council of Ministers of the Environment (CCME)” means the organizations of Canadian Ministers of Environment that sets guidelines for environmental protection across Canada such as the Canadian Water Quality Guidelines for the Protection of Freshwater Aquatic Life;

“Care and Maintenance” in respect of a mine, means the status of the facility when  the Licensee ceases production or Commercial Operation temporarily for an undefined period of time;

“Closure” means when an Operator ceases Operations at a facility without the intent to resume mining activities in the future;

“Collection Pond or Containment Pond” means a facility designed to temporarily contain runoff from areas impacted by mining activities, and site infrastructure as described  in the Water Management Plan

;

“Collection Pond No.1 or Control Pond No.1 or Containment Pond No. 1 (CP1)” means a contact water collection pond as described in the Water Management Plan;

“Commercial Operation” in respect of a mine, means an average rate of production equal to or greater than 10% of the design-rated capacity of the mine over a period of 90 consecutive days as defined in the Metal and Diamond Mining Effluent Regulations (SOR/2002-222 dated June 6, 2002 and as may be further amended from time to time).

“Construction” means any activities undertaken to construct or build any component of, or associated with, the development of the Meliadine Gold Project;

“Contact Water” means any Water that may be physically or chemically affected by mining activities;

“Dam Safety Guidelines” means the Canadian Dam Association (CDA) Dam Safety Guidelines

(DSG), (2007) or subsequent approved editions;

“Deposit” means the placement of waste rock, tailings or other materials on land or in Water;

“Diffuser” means an Effluent discharge pipeline within a water body designed to discharge and enhance mixing of Effluent in the Receiving Environment as described in the Water Management Plan;

“Discharge” means the release of any Water or Waste to the Receiving Environment;

“Dissolved Metals” means the suite of metals referred to in the Water Management Plan and in Group 2 of Table 1 – Monitoring Groups located in Schedule I of this Licence. Dissolved metals shall be analyzed on a filtered sample;

“Effluent” means treated or untreated liquid waste material that is discharged into the environment from the site water management facility such as a settling pond, tankfarm or a treatment plant;

“Emulsion Plant” means a facility designed for manufacturing of emulsion-based explosives, as indicated in the Explosives Management Plan;

“Engineer” means a professional engineer registered to practice in Nunavut in accordance with the Consolidation of Engineers and Geoscientists Act S. Nu 2008, c.2 and the Engineering and Geoscience Professions Act S.N.W.T. 2006, c.16 Amended by S.N.W.T. 2009, c.12;

“Engineered Structure(s)” means any facility, which was designed and approved by a Engineer;

“Final Discharge Point” in respect of an effluent, means an identifiable discharge point of a mine beyond which the operator of the mine no longer exercises control over the quality of the effluent as defined in the Metal and Diamond Mining Effluent Regulations (SOR/2002-222, June, 6, 2002  and as may be further amended from time to time);

“Fresh Water Intake” means the infrastructure required for extraction (pump system) of water as indicated in the Water Management Plan;

“Fuel Storage and Containment Facilities” means the facilities designed for the bulk storage  of fuel as described in the Hazardous Materials Management Plan;

“Geotechnical Engineer” means a professional engineer registered with the Association of Professional Engineers, Geologist and Geophysicists of Nunavut and whose principal field of specialization with the engineering properties of earth materials in dealing with man-made structures and earthworks that will be built on a site. These can include shallow and deep foundations, retaining walls, dams, and embankments;

“Grab Sample” means an undiluted quantity of material collected at a particular time and place that may be representative of the total substance being sampled at the time and place it was collected;

“Greywater” means the component produced from domestic use (i.e. washing, bathing, food preparation and laundering);

“Groundwater” means water that occupies pores and fractures in rock and soil below the ground surface in a liquid or frozen state;

“Hazardous Waste” means materials or contaminant which are categorized as dangerous goods under the Transportation of Dangerous Good Act (1992, c. 34, last amended on August 28, 2019 and as may be further amended from time to time) and/or that is no longer used for their original purpose and is intended for recycling, treatment, disposal or storage;

“High Water Mark” means the usual or average level to which a body of water rises at its highest point and remains for sufficient time so as to change the characteristics of the land (ref. Department of Fisheries and Oceans Canada, Operational Statement: Mineral Exploration Activities);

“Incinerator” means the dual chamber, high temperature facility designed with the capacity to service the camp as described in the Incineration Management Plan;

“Inspector” means an Inspector designated by the Minister under section 85 (1) of the Act;

“Interim Closure and Reclamation Plan” means a conceptual detailed plan on the Reclamation of mine components which will not be closed until the end of the operation, and operational detail for components which are to be progressively reclaimed throughout the mine life;

“Itivia Laydown Area” means the area designed for temporary storage of equipment and materials at Itivia Site in Rankin Inlet, as indicated in the Hazardous Materials Management Plan;

“Itivia Site Fuel Storage and Containment Facility” means the fuel storage and containment facility at Itivia Site in Rankin Inlet as described in the Hazardous Materials Management Plan;

“Itivia Site” means the Itivia Laydown Area and its facilities, including the fuel storage and containment facility, in Rankin Inlet as described in the Hazardous Materials Management Plan;

“Landfill” means a facility designed to dispose of non-salvageable, non-hazardous, non- putrescible solid wastes from the Construction, Operation, and Closure of the Project, as described in the Landfill and Waste Management Plan;

“Landfarm” means the engineered area designed to contain and treat hydrocarbon impacted sediment and soil using bioremediation as described in the Landfarm Management Plan;

“Licence” means this Type “A” Water Licence 2AM-MEL1631, issued by the Nunavut Water Board in accordance with the Act, to Agnico Eagle Mines Limited for the Meliadine Gold Project. Type “B” Water Licenses 2BW-MEL1525 and 8BC-MEL1516 are now amalgamated within File No: 2AM-MEL1631;

“Licensee” means to whom the Licence 2AM-MEL1631 is issued to or assigned;

“Maximum Average Concentration” means the average concentration of any four consecutively collected samples taken from the identical sampling location and taken during any given timeframe;

“Meliadine Security Management Agreement” means the agreement between Kivalliq Inuit Association, (KIA), Agnico-Eagle Mines Limited (Proponent) and Her Majesty Queen in Right of Canada as represented by the Minister of Indigenous and Northern Affairs Canada (the Minister), signed by KIA, AEM, and by the Minister that applies with respect to the Proponent’s Meliadine gold mine project.

“Metal Leaching” means the mobilization of metals into solution under neutral, acidic or alkaline conditions;

“Mine Water” means any water, including groundwater, that is pumped or flows out of any underground workings or open pit;

“Minister” means the Minister of Northern Affairs Canada;

“Modification” means an alteration to a physical work that introduces a new structure or eliminates an existing structure and does not alter the purpose or function of the work;

“Monitoring Program” means the program to collect data on surface water and groundwater quality to assess impacts to the environment of an appurtenant undertaking;

“Monthly” means, in the context of monitoring frequency, one sampling event occurring every thirty (30) days with a minimum of twenty one (21) days between sampling events;

“Nunavut Agreement” (NA) means the “Agreement Between the Inuit of the Nunavut Settlement Area and Her Majesty the Queen in Right of Canada,” including its  preamble and schedules, and any amendments to that agreement made pursuant to it;

“Nutrients” means the suite of parameters referred to in Water Management Plan, and in Group 2 of Table 1 – Monitoring Groups located in Schedule I of this Licence;

“Operation or Operations” means the entire set of site activities (excluding construction, care and maintenance, and decommissioning activities) associated with mining, processing and recovery of gold;

“Operator” means the person who operates, has control or custody of, or is in charge of a mine or recognized closed mine;

“Ore Stockpile” means the above-ground facility designated for the temporary storage of ore to be processed in the mill as indicated in the Ore Storage Management Plan;

“Progressive Reclamation” means actions that can be taken during mining Operations before permanent closure, to take advantage of cost and operating efficiencies by using the resources available from mine Operations to reduce the overall reclamation costs incurred. It enhances environmental protection and shortens the timeframe for achieving the reclamation objectives and goals;

“Project” means the Meliadine Gold Project as outlined in the public registry for the Nunavut Impact Review Board (NIRB) File No. 11MN034 and for the Nunavut Water Board File No: 2AM-MEL1631, and 2BW-MEL1525 and 8BC-MEL1516 that are now amalgamated within File No: 2AM-MEL1631;

“Quarry or Quarries” means the area of surface excavation for extracting rock material for use as construction materials in the development of site infrastructure and facilities;

“Quality Assurance / Quality Control (QA/QC)” Quality Assurance means the system of activities designed to better ensure that quality control is done effectively; Quality Control means the use of established procedures to achieve standards of measurement for the three principle components of quality: precision, accuracy and reliability;

“Receiving Environment” means both the aquatic and terrestrial environments that receive any discharge resulting from the Project;

“Reclaim Water” means the water pumped from the Control Pond No.1 (CP1) or the Water Treatment Plan to the mill for reuse;

“Reclamation” means the process of returning a site to the pre-development or natural state, or a state that prevents environmental impacts or threats to human health and safety.

“Regulations” means the Nunavut Waters Regulations (SOR/2013/669 18th April, 2013);

“Seepage” means any Water that drains through or escapes from any structure designed to contain, withhold, divert or retain Water or Waste. Seepage also includes any flows that have emerged through open pits, runoff from waste rock storage facilities, ore stockpile areas, quarries, landfill or landfarm areas;

“Sewage” means all toilet wastes;

“Sewage Treatment Plant” means the structure designed for the treatment of Sewage as indicated in the Water Management Plan;

“Site Specific Water Quality Objectives (SSWQO)” means a numerical concentration established as a target value which has been established for specified waters;

“Sump” means a structure or depression that collects, controls, and filters liquid waste before it is released to the environment. This structure should be designed to prevent erosion while allowing percolation of liquid waste;

“Surface Drainage” means contact Waters resulting from the flow over, through or out of an Operation area and is collected as described in the Water Management Plan;

“Tailings Storage Facility” means the engineered facility designated as a Tailings Storage Facility to receive dry stack tailings as described in the Mine Waste Management Plan;

“Open Pits” mine deposits to be developed using a traditional open-pit mining method and underground mining as described in the Mine Plan;

“Total Metals” means the suite of metals referred to in the Water Management Plan, and in Group 2 of Table 1 – Monitoring Groups located in Schedule I of this Licence.  Total metals shall be analyzed on an un-filtered sample;

“Underground” means the underground workings at the Project as described in the Mine Plan;

“Undertaking” means an undertaking in respect of which water is to be used or waste is to be deposited, of a type set out in Schedule I of the Regulations;

“Use”  in relation to Waters, means Use as defined in S.4 of the Act;

“Waste” means, Waste as defined in section 4 of the Act;

“Waste Disposal Facilities” means all site infrastructure designed for the disposal of waste including the Landfill, Incinerator, Landfarm, Sewage Treatment Plant, Ore Storage Facilities, and Waste Rock Storage Facilities;

“Waste Rock” means all rock materials, except ore and tailings, that are produced as a result of mining operations.;

“Waste Rock Storage Facility” means the engineered structure or structures designed for the placement of Waste Rock as described in the Mine Waste Management Plan;

“Wastewater” means the water generated by site activities or originates on-site that requires treatment or any other water management activity;

“Water or Waters” means water as defined in S.4 of the Act;

“Water Licence Application” for the purposes of this Licence includes the totality of the NWB and NIRB Public Registries established as a result of the filing of the application dated March 8, 2010, replaced by the Application dated May 15, 2015 and the Amendment Application filed August 2020; and, including Supporting Documents, and Technical Meeting Information supplemental documents;

“Water Supply Facilities” means the Fresh Water Intake, the Reclaim Water system and associated infrastructure;

“Water Treatment Plant (WTP)” means the facilities designated for the treatment of water as described in the Water Management Plan;

“Weekly” means, in the context of monitoring frequency, one sampling event occurring every 7 days with a minimum of 5 days between sampling events.

# Schedule B:	General Conditions

The Annual Report referred to in Part B, Item 2, shall include:

# CONSTRUCTION

- For structures constructed to withhold Water or Waste:

  - An overview of methods and frequency used to monitor deformations, Seepage and geothermal responses;

  - A comparison of measured versus predicted performance;

  - A discussion of any unanticipated observations including changes in risk and mitigation measures implemented to reduce risk;

  - As-built drawings of all mitigation works undertaken;

  - Any changes in the design and/or as-built condition and respective consequences of any changes to safety, water balance and water quality;

  - Data collected from instrumentation used to monitor earthworks and an interpretation of that data;

  - A summary of maintenance work undertaken as a result of settlement or deformation of dikes, dams and berms;

  - The daily, monthly and annual flow volumes of any watercourse diverted during Construction activities and

  - The daily, monthly and annual quantities of Seepage from dikes, dams and other structures in cubic metres.

# WATER

- Monthly and annual volume of fresh Water obtained from Meliadine Lake.

- Monthly and annual volume of fresh Water transferred to Meliadine Lake as a result of dewatering activities.

- Monthly and annual volume of fresh Water obtained from Meliadine River for road dust suppression activities.

- Summary of reporting results for the Water Balance and Water Quality model as required in Part E Item 14.

# WASTE

- Geochemical monitoring results including:

  - Operational acid/base accounting and paste pH test work used for waste rock designation (PAG and NPAG rock);

  - As-built volumes of waste rock used in construction and sent to the Waste Rock Storage Facilities with estimated balance of acid generation to acid neutralization capacity in a given sample as well as metal toxicity;

  - All monitoring data with respect to geochemical analyses on site and related to roads and quarries;

  - Leaching observations and tests on pit slope and dike exposure;

  - Any geochemical outcomes or observations that could imply or lead to environmental impact;

  - Geochemical data associated with tailings, cyanide leach residue, and bleed from the cyanide destruction process including an interpretation of the data; and

  - Results related to the Borrow pits/ Quarries and roads, including the All-weather Access Road.

- An update on the current capacity of the Tailings Storage Facility.

- Summary of quantities and analysis of Seepage and runoff monitoring from the Landfill, Landfarm, Waste Rock Storage Facilities, Borrow pits and Quarries.

- A summary report of all general waste disposal activities including monthly and annual quantities in cubic metres of waste generated and locations of disposal.

- Report of Incinerator test results including the materials burned and the efficiency of the Incinerator as they relate to Water and the deposit of Waste into Water.

# SPILLS

- A list and description of all unauthorized discharges including volumes, spill report line identification number and summaries of follow-up action taken.

# MODIFICATIONS

- A summary of modifications and/or major maintenance work carried out on all Water and waste related structures and facilities.

# MONITORING

- The results and interpretation of the Monitoring Program in accordance with Part D and Part I and Schedule I.

- The results of monitoring related to the Environmental Management and Protection including:

  - Aquatic Effects Monitoring Program;

  - Metal and Diamond Mining Effluent Regulation (MDMER) Monitoring;

  - Mine site Water quality monitoring, including groundwater monitoring; and

  - Visual AWAR Water quality monitoring.

# CLOSURE

- A summary of any progressive closure and reclamation work undertaken including photographic records of site conditions before and after completion of operations, and an outline of any work anticipated for the next year, including any changes to implementation and scheduling.

- A summary of on-going field trials to determine effective capping thickness for the Tailings Storage Facility and Waste Rock Storage Facilities for the purpose of long term environmental protection.

- An updated estimate of the current restoration liability based on Project development monitoring, results of restoration research and any changes or modifications to the Appurtenant Undertaking.

# PLANS/REPORTS/STUDIES

- A summary of any studies requested by the Board that relate to Water use, Waste  disposal or Reclamation, and a brief description of any future studies planned.

- Where applicable, revisions as Addendums, with an indication of where changes have been made, for Plans, Reports, and Manuals.

- An executive summary in English and Inuktitut of all updated plans, reports, or studies conducted under this Licence.

# GENERAL

- A summary of actions taken to address concerns or deficiencies listed in the inspection reports and/or compliance reports filed by an Inspector.

# OTHER

- A summary of public consultation and participation with local organizations and the residents of the nearby communities, including a schedule of upcoming  community events and information sessions.

- Any other details on Water use or Waste disposal requested by the Board by November 1st of the year being reported.

# Schedule D:	Conditions Applying to Construction

- The Construction Summary Report referred to in Part D Item 3 shall include:

  - All final design and construction drawings shall be stamped and signed by an Engineer.

  - Site specific data and analysis, including Geochemical analysis of waste rocks and fills, demonstrating their Non Acid Rock Drainage and Non Metal Leaching characteristics, to support the design and management decisions

  - A summary of construction activities including photographic records before, during and after construction;

  - As-built drawings:

  - Documentation of field decisions that deviate from original plans and any data used to support these decisions;

  - Discussion of mitigation measures implemented during construction and effectiveness;

  - Construction monitoring summary including Monitoring undertaken in accordance with Part D;

  - Blast vibration monitoring for activities carried out in close proximity to fish bearing waters;

  - Monitoring for sediment release from construction areas; and

  - Monitoring and reporting on use of water to manage dust emissions from crushing and construction activity.

# Schedule I:	Conditions Applying to General and Aquatic Effects Monitoring

- The Annual Geotechnical Inspection referred to in Part I Item 14 shall include:

  - Dikes;

  - Berms;

  - Collection Ponds;

  - Channels;

  - Jetties;

  - Water Intake causeway;

  - All Weather Access Road (AWAR) and site roads, in particular bridges and culverts;

  - Sumps;

  - Industrial Pads;

g.	Open Pits;

- Ore Stockpiles;

- Underground portals;

- Tailings Storage Facility;

- Waste Rock Storage Facilities;

- Landfill;

- Landfarm;

- Fuel Storage and Containment Facilities at Meliadine site and Itivia site;

- Geotechnical instrumentation and associated monitoring data; and

- A description of geophysical and permafrost conditions at the project site.

TABLE 1 – MONITORING GROUP

| Group | Parameters |
| --- | --- |
| 1 | pH, turbidity, hardness, total alkalinity, sodium, magnesium, potassium, calcium, fluoride, silicate, chloride, fluoride, sulphate, total dissolved solids (TDS; calculated(a,b)), total suspended solids (TSS), total cyanide, ammonia nitrogen, nitrate, nitrite, phosphorus, orthophosphate, Total Metals (aluminum, arsenic, barium, cadmium, chromium, copper, iron, lead, manganese, mercury, molybdenum, nickel, selenium, silver, thallium, and zinc). |
| 2 | Total and Dissolved Metals: aluminum, antimony, arsenic, barium, beryllium, boron, cadmium, chromium, copper, iron, lead, lithium, manganese, mercury, molybdenum, nickel, selenium, silver, strontium, thallium, tin, titanium, uranium, vanadium, and zinc. Nutrients: ammonia-nitrogen, total Kjeldahl nitrogen, nitrate-nitrogen, nitrite- nitrogen, orthophosphate, total phosphorus, total organic carbon, dissolved organic carbon, and reactive silica. Conventional Parameters: bicarbonate alkalinity, chloride, carbonate alkalinity, turbidity, conductivity, hardness, calcium, potassium, magnesium, sodium, sulphate, pH, total alkalinity, TDS (calculated(a,b)), TSS, total cyanide, free cyanide, and weak acid dissociable (WAD) cyanide. |
| 3 | MDMER parameters: total cyanide, arsenic, copper, lead, nickel, zinc, radium- 226, TSS, pH, sulphate, turbidity, and aluminum. MDMER additional requirements: Effluent volumes and flow rate of discharge, Acutely Lethality tests and environmental effects monitoring (EEM). |
| 4 | Total arsenic, total copper, total lead, total nickel, TSS, ammonia, benzene, toluene, ethylbenzene, xylene, total petroleum hydrocarbons (TPH), and pH. |
| Full Suite | Group 2, TPH, and turbidity. |
| Flow | Flow data-logger. |
| Field measurements | Field pH, specific conductivity, dissolved oxygen, and temperature. |

MDMER - Metal and Diamond Mining Effluent Regulations (SOR/2002-222).

  - Standard Methods (Method 1030E, APHA 2012

  - TDSCalc (mg/L) = (0.6 x Total Alkalinity as CaCO3) + Sodium + Magnesium + Potassium + Calcium + Sulfate + Chloride + Nitrate + Fluoride + Silicate

Where: Nitrate is the NO3- anion (multiply nitrate as nitrogen result by 4.427); Silicate is the SiO32- anion (multiply reactive silica as SiO2 result by 1.266)

TABLE 2 – MONITORING PROGRAM

| Station | Description | Phase | Monitoring Parameters | Frequency |
| --- | --- | --- | --- | --- |
| MEL-D-1 to TBD | Dewatering: Water transferred from lakes to Meliadine Lake during dewatering of lakes | Construction | As defined in the Water Management Plan referred to in Part D, Item 12 | Prior to discharge and Weekly during discharge |
|  |  |  | Volume (m3) | Daily during periods of discharge |
| MEL-SR-1- TBD | Surface Runoff – runoff downstream of Construction areas at Meliadine Site and Itivia Site, Seeps in contact with the roads, earthworks and any Runoff and/or discharge from borrow pits and quarries | Construction, and Operations | As defined in the Water Management Plan referred to in Part D, Item 18 and Part I, Item 11 | Prior to Construction, Weekly during Construction |
|  |  |  | Group 1 | Monthly during open water or when water is present upon completion |
| MEL-11 | Water intake from Meliadine Lake | Construction, Operation, and Closure | Full Suite | Monthly during periods of intake |
|  |  |  | Volume (m3) | Daily during periods of intake |
| MEL-12 | Water treatment plant (pre- treatment) coming from CP1, off the pipe and not in the pond | Construction (prior to release), Operations, and Closure | Group 1 | Monthly during periods of discharge |
| MEL-13(a) (and AEMP Stations) | Mixing zone in Meliadine Lake, Station 1; and MDMER exposure stations for final discharge point within mixing zone | Construction (prior to release), Operations, and Closure | Full Suite, Group 3 (MDMER) | Monthly during periods of discharge |
| MEL-14 | Water treatment plant from CP-1 (post-treatment), end of pipe (before offsite release) in the plant before release. | Construction (upon effluent release), Operations, and Closure | Full Suite, Group 3 | Prior to discharge and Weekly during discharge |
|  |  |  | Volume (m3) | Daily during periods of discharge |
|  |  |  | Acute Lethality | Once prior to discharge and Monthly thereafter |
| MEL-15 | Local Lake E-3 | Operations, and Closure | Group 2 | Bi-annually during open water |
| MEL-16 | Local Lake G2 | Construction, Operations, and Closure | Group 2 | Bi-annually during open water |
| MEL-17 | Local Pond H1 | Construction, Operations, and Closure | Group 2 | Bi-annually during open water |
| MEL-18 | Local Lake B5 | Construction, Operations, and Closure | Group 2 | Bi-annually during open water |
| MEL-19 | CP-2, Collection of drainage from WRSF3 | Construction, Operations, and Closure | Group 1 | Monthly during open water or when Water is present |
| MEL-20 | CP-3, Collection of drainage from dry stacked tailings | Operations, and Closure | Group 1 | Monthly during open water or when water is present |
| MEL-21 | CP-4, Collection of drainage from WRSF1 | Operations, and Closure | Group 1 | Monthly during open water or when water is present |
| MEL-22 | CP-5, Collection of drainage from WRSF1 and WRSF2 | Construction, Operations, and Closure | Group 1 | Monthly during open water or when water is present |
| MEL-23 | CP-6, Collection of drainage from WRSF3 | Construction, Operations, and Closure | Group 1 | Monthly during open water or when water is present |
| MEL-24 | Seepage from the Landfill between the landfill and Pond H3 | Construction, Operations, and Closure | Group 1 | Monthly during open water or when water is present |
| MEL-25 | Secondary containment area at the Itivia Site Fuel Storage and Containment Facility | Construction, Operation, Closure | Group 4, Volume (m3) | Prior to discharge or transfer of Effluent |
| Monitoring Legend: Green - Regulated; Blue - General Aquatic; Red - Verification |  |  |  |  |  |
| Regulated Monitoring occurs at Monitoring Program Stations in licences or regulations. It includes discharge limits that must be achieved to maintain compliance with water licence or regulation (i.e., Metal Mining Effluent Regulations). Enforcement action may be taken if discharge limits are exceeded. |  |  |  |  |  |
| General Aquatic Monitoring is subject to compliance assessment to confirm sampling is carried out using established protocols, including quality assurance/quality control provisions, and addresses identified issues. General monitoring is subject to change as directed by an Inspector, or by the Licensee, subject to approval by the NWB. |  |  |  |  |  |
| Verification Monitoring Program to be carried out for operational and management purposes by Licensee. Monitoring parameters may vary between locations. Monitoring parameters and locations are internal for Licensee. |  |  |  |  |  |

Notes: as per Metal Diamond Mining Effluent Regulations (MDMER), samples for Effluent characterization and Receiving Environment must be collected quarterly or at least one month apart while Effluent is being deposited.

a Sampling may not occur during break-up (June) CP - Collection Pond;

WRSF - Waste Rock Storage Facility.

APHA ((American Public Health Association). 2012. Standard Methods for the Examination of Water and Wastewater, 22nd Edition, with updates to 2015. Washington, DC, USA.