	
[image: image1.png]



	MIRAMAR HOPE BAY LTD.
300 - 889 Harbourside Drive, North Vancouver, B.C. V7P  3S1 Canada

Phone 604-985-2572   Fax 604-980-0731


11 August 2004




Via e-mail only:


Mr. David Hohnstein

Technical Advisor Mining

Nunavut Water Board,

P.O. Box 119

Gjoa Haven, NU, X0E 1J0

Dear  Mr. Hohnstein,

Re:
July 2004  Monthly Report for Water License: NWB1BOS0106

Activities at the Boston camp during the months of July focused on activities related to the Boston Deeps exploration program.  The camp was fully operational during the month of July and the population for the report period was approximately 50 personnel.  These numbers are expected to remain consistent throughout the 2004 program. 

Water for all purposes is pumped from one pumping station, that being 1652-1a.  The licence allows for water use of 150 cubic metres per day.   There was no activity underground and hence nothing to report as per the attached table.

	Intake Site
	Particulars
	June , 2004
	July, 2004
	Year-to-Date (YTD)

	1652-1a
	Camp & Domestic Use
	 230 m3
	240 m3
	1,136 m3

	1652-1a
	Underground Operations – Inactive
	0 m3
	0 m3
	0 m3


	Outflow  Site
	Particulars
	Total Waste Discharged into Spyder (Aimaokatak) Lake



	1652-2
	Underground Operations - Inactive
	0 m3


The following table provides results of water quality samples collected during the report period as per the SNP requirements of this licence.  Analyses results for the report period are provided in the following table:

	Parameters
	SNP #1652-3
	SNP#1652-4
	SNP #1652-5

	
	Sewage-End

of Pipe
	Sewage effluent point discharge into Spyder Lake 
	Sewage effluent point discharge into Spyder Lake

	Sample Date
	July 8, 2004
	July 8, 2004
	July 8, 2004


	Physical Tests
	Results
	
	

	Hydrogen ion – field pH
	7.17
	7.79
	7.85

	Total Suspended Solids (mg/L)
	190
	4
	5

	Field Sample Temperature (oC)
	19
	5
	5

	Bacteriological Tests
	
	
	

	Coliforms – Faecal (CFU/100mL)
	510,000
	<1
	<1

	Extractable
	
	
	

	Oil and Grease (No Visible Sheen)
	No-Visible Sheen
	No visible sheen
	No visible sheen

	Organic Parameters
	
	
	

	BOD5 (mg/L)
	380
	5
	4

	Nutrients
	 
	
	

	Orthophosphate as P  (mg/L)
	11.1
	<0.002
	<0.002

	Phosphate, Total (mg/L)
	17.7
	<0.01
	<0.01

	Subcontracted Nutrients
	 
	
	

	Kjeldahl Nitrogen, Total
	83
	
	

	Nitrate+Nitrite as N (mg/L)
	<2
	<2
	<2


There were no reportable spills for the period reported.  

If you have any questions related to this matter, please do not hesitate to contact the undersigned on the above numbers or 1-780-975-2550, or you can contact me by e-mail at hugh_r_wilson@hotmail.com; or hwilson@miramarmining.com or Matthew Kawei, Senior Environmental Coordinator at 604-677-0617 or by email on mkawei@miramarmining.com. . 

Sincerely, 

Miramar Hope Bay Ltd.

Hugh R. Wilson,

Manager, Environmental Affairs 

CC:
Tim Canam  Project Manager/Boston- MHBL


Matthew Kawei : Senior Environmental Coordinator - MHBL


D. Fonseca: MHBL Vancouver












Page 1 of 2

_949745930

