Water usage for the Barrow Project under NWB permit 2BE-BAR0712 occurred in 2007 and 2008 for diamond drilling purposes. A total of 10 holes were completed with 5 holes occurring in each year.

The table below indicates the drill hole location as well as water source location. Water source during ice-based drilling was from directly below the drill rig and land-based drilling used nearby lakes or streams. In both cases a supply pump was used to deliver the water with the suction hose having a screen over the end to prevent fish entrapment.

| Hole Number | Hole | Hole | Topo | Water Source | Water Source |
| --- | --- | --- | --- | --- | --- |
|  | Latitude | Longitude |  | Latitude | Longitude |
| DDH-UP-07-01 | 68° 17' 21.5" | 89° 36' 5.8" | Land | 68° 17' 22" | 89° 35' 57" |
| DDH07-UP012 | 68° 17' 32.4" | 89° 36' 11.3" | Land | 68° 17' 22" | 89° 35' 57" |
| DDH07-UP020 | 68° 15' 36.8" | 89° 38' 15.2" | Land | 68° 15' 37" | 89° 38' 15" |
| DDH07-UP023-1 | 68° 21' 18.9" | 89° 48' 45.3" | Land | 68° 21' 19" | 89° 48' 45" |
| DDH07-UP027-1 | 68° 21' 20.1" | 89° 49' 38.8" | Land | 68° 21' 20" | 89° 49' 39" |
| DDH08-UP01-1 | 68° 21' 0.6" | 89° 46' 27.3" | Ice | 68° 21' 0.6" | 89° 46' 27.3" |
| DDH08-UP02-1 | 68° 15' 50.9" | 89° 49' 10" | Ice | 68° 15' 50.9" | 89° 49' 10" |
| DDH08-UP03-1 | 68° 20' 46.3" | 89° 49' 11.5" | Ice | 68° 20' 46.3" | 89° 49' 11.5" |
| DDH08-UP04-1 | 68° 15' 17.3" | 89° 31' 14.8" | Ice | 68° 15' 17.3" | 89° 31' 14.8" |
| DDH08-UP05-1 | 68° 17' 41.5" | 89° 39' 46.7" | Ice | 68° 17' 41.5" | 89° 39' 46.7" |

Drill water return was processed through the whiffle system which flocculates drill cutting returns causing them to settle to the bottom of the water tank so that the drill water can be recirculated resulting in a vastly lower amount of fresh water needed. The “sludge” at the bottom of the tank can then be periodically drained out.

Drill waste (cutting returns) generated during the 2007 land-based program was deposited into a natural depression proximal to the drill rig. During the 2008 ice-based drilling the drill waste was collected in sausage type poly-bags and transported to town and deposited in the local land fill.

Water usage for each of the years was 4 cubic meters.