FERGUSON LAKE PROJECT

STATEMENT OF ASSIGNEE’S CAPACITY TO IMPLEMENT THE CONDITIONS OF THE WATER LICENCE AND 2018 WORK PLAN
INTRODUCTION
This work plan is presented by Canadian North Resources and Development Corp. (Assignee), a private Ontario incorporated company, which purchased the Ferguson Lake Property and Project from Starfield Resources Inc. on June 8, 2018, as support for the assignee’s capacity to implement the conditions of the water license.  The plan provides a schedule for the implementation of plans, manuals and reports on the Ferguson Lake Property and Project.
PROPERTY DESCRIPTION AND LOCATION
The Ferguson Lake property consists of 29 active mineral claims and 10 mining leases comprising an area of 28,128 hectares (69,476 acres) 100% held by Canadian North Resources and Development Corporation (CNRD). The property is located in the Kivalliq region of southern Nunavut Territory some 240 kilometres west of Rankin Inlet and 160 kilometres south‐southwest of Baker Lake. Ferguson Lake, central to the large property area, is midway between Yathkyed and Qamanirjuaq Lakes. 

The property encompasses an area which measures approximately 90 kilometres in an east‐west direction and 70 kilometres north‐south, and consists of three contiguous claim blocks plus one block of contiguous mining leases extending across and south of Ferguson Lake between latitudes 62°30’ and 62°55’ North and longitudes 96°10’ and 97°30’ West in NTS map‐areas 65I/10‐16 (UTM coordinates 6,945,000 – 6,978,000N, 585,000 –625,00E – Zone 14).  The mining leases consist of 11,456 hectares (23,935 acres), cover the Ni-Cu-Co-Pd-Pt Ferguson Lake Deposit, and are located predominately on Kivalliq Inuit Association (KIA) surface rights owned land (RI-24, RI-25, RI-26 and RI-27).  

In addition to the above, in February 2014 five Prospecting Permits totaling 232,500 acres (105,682 hectares) were applied and issued to CNRD listed as follows:
Claim#   Area(ha)  Status    Issue Date  
Cancel Date  
 Owner


    
	P-8424
	18817.88
	ACTIVE
	1
	2014-02-01
	2019-02-01
	Canadian North Resource and Development Corporation (100%)

	P-8425
	18615.54
	ACTIVE
	1
	2014-02-01
	2019-02-01
	Canadian North Resource and Development Corporation (100%)

	P-8426
	18817.88
	ACTIVE
	1
	2014-02-01
	2019-02-01
	Canadian North Resource and Development Corporation (100%)

	P-8427
	18817.88
	ACTIVE
	1
	2014-02-01
	2019-02-01
	Canadian North Resource and Development Corporation (100%)

	P-8428
	18615.54
	ACTIVE
	1
	2014-02-01
	2019-02-01
	Canadian North Resource and Development Corporation (100%)


The following Commercial, Land Use and Right of Way permits, enabling work, water use, and travel to be conducted over the mining leases, claims, and Prospecting Permit areas, are issued by the Kivalliq Inuit Association (KIA), Nunavut Water Board (NWB), and by Aboriginal Affairs and Northern Development Canada (AANDC): 
· KVCL305H27



Expires July 22, 2022
· KVCA08Q17



Expires September 8, 2019
· KVRW06F09



Expires October 17, 2019
· 2BE-FER1318 TYPE “B”

Expires December 11, 2018

· N2013X0023



Expires March 17, 2019
BACKGROUND AND PROJECT DESCRIPTION
Starfield Resources Inc. (Starfield) had been exploring a significant nickel, copper, palladium and platinum group metals deposit located in an area of Inuit Owned Lands at Ferguson Lake in the Kivalliq Region of Nunavut until 2012.  Until 2007, exploration had been based out of the privately owned Ferguson Lake Fishing Lodge, located on an island in Ferguson Lake.  In 2006, Starfield began moving the exploration camp from the island to the mainland.

The New Camp location on the west side of Ferguson Lake is safer and provides more convenient access to the current exploration areas, allowing for potential expansion as the project advances. As a result, this new location will serve the longer term needs of the project.

The infrastructure at the New Camp site include buildings for accommodation, offices and areas for fuel storage, core storage, equipment maintenance, core cutting, warehousing, etc.  Freshwater will continue to be drawn by pump from Ferguson Lake to the camp.

The airstrip on the Island at the Old Camp location was used for a short time in 2008 to support the exploration program while the new airstrip on the mainland was being constructed. This is in accordance with the KIA’s direction to the company to move from the Island.  The new airstrip, which was completed in 2008, is fully operational. 
Detailed environmental studies, conducted by Rescan Environmental Services Ltd. (Rescan) on behalf of previous owners Starfield, had been ongoing since 1999.  In 2008, Starfield expanded the environmental baseline studies significantly in the area of Ferguson Lake.  

After CNRD acquired of the Ferguson Lake project in June, 2013, the camp was re-opened that summer for rehabilitation and maintenance including the reactivation of the airstrip. During this time ERM Rescan completed a program of repair of the weather station and of sampling and analytical analysis of waters in the Ferguson Lake camp area focusing on standing water within the fuel berm.  The camp was inspected by the KIA in September, 2013.  Copies of the camp inspection report and ERM Rescan water testing report was forwarded by CNRD to the KIA, AANDC, and NWB.  The camp was kept closed throughout 2014. A Water Licence Inspection was conducted at the camp in July, 2014 by the AANDC. The inspection report was issued in February, 2015 which has been shared with the NWB and the KIA.  An inspection of the camp and area was completed by the KIA during the summer of 2015 for which the report has been issued which was shared with the NWB and AANDC.

PREVIOUS WORK
During 2016 and 2017, there was no activity on the Ferguson Lake Property and the camp remained closed.  An assessment report covering the 2015 exploration program was prepared and filed with the Nunavut mining recorder.  Camp and airfield insurance has continued to be maintained throughout this time
During 2015, the Ferguson Lake camp was re-opened temporarily during parts of July and August during which a sampling survey of both the fuel berm waters and waste storage ground area was completed by ERM Rescan for which a report has been issued and copies were provided to the KIA, NWB and AANDC.  During 2015, updated Abandonment and Restoration Plan and Spill Contingency Plan were prepared and submitted to both the KIA and AANDC, recently renamed Indigenous and Northern Affairs Canada.  

While the camp was open maintenance work was completed on the buildings, equipment and airfield. The airfield is presently in operative condition. A helicopter supported reconnaissance geology and rock/till sampling program was completed during the summer with the main purpose of evaluating areas of favorable Ni-Cu-PGE, Au, and diamond potential situated mostly outside the main Ferguson Lake mining leases focusing on the areas of the Prospecting Permits and claims.  In addition, during the program a ground magnetometer and VLF survey was completed over an area covering the eastern margin of the mining lease 4931 and the southwestern part of Prospecting Permit P-8428.  During the program seven claims, totaling 6,982 hectares, adjoining the eastern and northern boundaries of the mining leases were staked. 

No significant exploration on the property was conducted from 2012-2014. The camp has been closed and secured during the winters since 2011.  After the water use license expired on July 1, 2012; an inspection of the site reported items of non-compliance of Act or License in the storage of solid waste and fuel storage in the camp area.  The camp was opened for three weeks during July and August, 2012 in order to complete required maintenance of the site and to address deficiencies identified in Part H item 3 of the Act from the camp inspection.  

During the 2012 work program, Rescan prepared, compiled and submitted the required documents to the Nunavut Water Board for the Renewal of the water use license including an updated plan for care and maintenance dated November, 2012.  In addition, a program of sampling, analytical analysis and discharge of waters within the fuel storage area was completed in October, 2012. During the 2012 work program, remediation work was undertaken in the area of the old camp site with the placement of contaminated soil into barrels and clean-up of scrap material and barrels.  The material was then transported to the site of the new camp to be bundled and removed by snow cat.  A renewed Class B water use licence for the new Ferguson Lake Camp was issued in 2014.  
Exploration work in 2011 included; prospecting, claim staking, geochemical sampling and drilling. A total number of 9 samples were collected for analysis. The 2011 drill program consisted of three core holes totaling 1,866 m.  One of the holes was drilled within the known deposit while two were drilled southwest of the West Zone to extend and further evaluate deep massive sulphide mineralization and / or investigate coincident geochemical and geophysical anomalies. 

Exploration work in 2010 included; prospecting, claim staking, sampling and drilling.  A total number of 23 samples were collected for analysis. The drill program consisted of four holes drilled in the Y Lake area approximately 20 km south of the camp and two holes drilled approximately 6 km southwest of the camp. This drilling totaled 1,126 metres.  In 2009, Starfield drilled one core hole totaling 407 m east of Y Lake, while Thanda Resources, a joint venture partner in diamond exploration at the time, drilled seven shallow holes to test geophysical anomalies in their diamond exploration program.

In 2006/2007, Starfield, at the request of the KIA, began construction of the new (existing) camp. In 2010, only minor maintenance of the new camp, which has enough beds to house 70 people, was undertaken.  In 2008, Starfield quarried material to construct an airstrip at the new camp location.  The airstrip was completed and fully operational by the fall of 2008. The airstrip is 825 m in length X 30 m wide. 

Environmental baseline data was collected during the period 1999- 2008.  In 2007, the study was expanded and a detailed wildlife monitoring program was conducted.  In 2008, this program was expanded upon and a comprehensive environmental baseline study was undertaken. Due to budgetary constraints, no environmental or wildlife monitoring was undertaken during the 2009 and 2010 programs.
PLANNED ACTIVITIES FOR 2018
A) Exploration

The undertaking of the 2018 exploration program on the property under the management of CNRD will be dependent upon budget availability.  It is planned that the Ferguson Lake Camp will be re-opened temporarily during August 2018 to conduct necessary maintenance work on the camp infrastructure, vehicles and airfield.  During this time, there will also be some re-sampling of stored drill core and mineral showings in the surrounding area on CNRD active claims and Prospecting Permits.  A diamond drilling program is being considered on the property during the winter of 2018-19.

The Department of Indigenous and Northern Affairs Canada (INAC) has informed CNRD of the reported discovery by a community member of a rock crusher about 40 km west of Rankin Inlet which had been abandoned by Starfield Resources Inc. during the winter of 2009-2010 while attempting to haul overland to the Ferguson Lake Camp under permit 2BE-FER0712 issued to Starfield.  The location and discovery has not been verified as yet by INAC or CNRD.  At the request of INAC,  a response plan has been submitted by CNRD to confirm the discovery of the equipment and recover if possible.  The INAC Land Use Inspection Form and CNRD plan are attached to this Work Plan as information for the Kivalliq Inuit Association.  

Geophysical Surveys

Borehole geophysical surveys may be planned during the winter of 2018-19 in conjunction with any diamond drill program. 

Sampling

Follow-up helicopter supported sampling is being considered for 2018 which would include the collection of rock core and metallurgical samples. 

Drilling

Prior to any drilling, a detailed review of drill core geochemistry, core logging, resource evaluation, and geological and geophysical models for both the nickel, copper, palladium and platinum group metals deposition on the property will continue to be undertaken.  At this time, an up to 2,000 meter drill program is being considered for the winter of 2018-19 with a focus on exploration for potentially high grade zones located directly underneath Ferguson Lake at the Central Zone at depths of greater than 150 metres.  As in the past, this program will be supported from the camp via snowmobile and ATV (under snow and frozen ground conditions if drilling is contemplated for the following winter).
Bulk Sampling

A mini-bulk sample was taken during the 2011 program, and the bulk sample site reclaimed. Additional bulk sampling is not anticipated in the 2018 program. This is located at the diamond drill staging area near the bulk sampling site where some iron oxide weathering and erosion was noted in the 2014 Water License inspection report. Based upon investigation of the site in 2015, this weathering is similar in appearance to natural iron sulphide gossans observed in airborne images in the same area captured prior to drilling and sampling activity.   
B) New Camp Construction and Maintenance 
Significant work was done in 2008 to the New Camp including; construction of a new airstrip, installation of 3 fuel bladders and additional sleep quarters (see attached camp photo).  During 2011, substantial needed camp maintenance was undertaken and the fuel bladders decommissioned. No major construction activities are planned for 2015, however, routine camp and airfield maintenance will continue.  The ROW land use permit (N2013X0023) over Crown Land was extended to 2019 by AANDC (INAC) so the removal of scrap steel and empty fuel drums is delayed at least until the winter of 2018-19, which will be completed overland by snow train, dependent upon favorable weather and ground conditions, and budget availability.
C) Engineering

In the spring of 2008, Starfield completed a Scoping Study and began the next phase needed to undertake a feasibility study of the project.  In 2008, BRASS Engineering, on behalf of Starfield, travelled to Ferguson Lake to begin investigations for a slurry pipeline routing and design. The planned slurry pipeline would transport a sulphide concentrate slurry from the Ferguson Lake mill (crushing, grinding and flotation) to the hydrometallurgical processing plant location to Arviat.

In 2009, Starfield continued to investigate the feasibility of the slurry pipeline and will begin to gather data on potential port locations and a processing plant facility.  Future work will be coordinated with the Environmental Baseline studies and the socioeconomic studies.  A preliminary route for the slurry pipeline was mapped out by an engineering company. There are no additional studies related to the slurry pipeline route, the port facility or the processing plant facility planned.  In 2014, alternate development scenarios were being investigated by CNRD focusing on a metallurgical review and tests of the technical feasibility and economics of extraction and recovery of platinum group metals with the base metals from the Ferguson Lake resource.  In 2016, follow-up metallurgical tests on the mineralized material were completed creating by flotation separate high-grade Cu and a low-grade Ni-Cu bulk sulphide concentrates.
D) Hydrocarbon Assessment

Starfield conducted a hydrocarbon sampling program in 2007.  This sampling program was expanded in 2008 and the results of the sampling programs were used to develop a Remedial Action Plan.  A full reclamation plan for the old island camp was presented and submitted to the KIA, and this program is waiting for final approval.  Material from the 2012 clean-up of the old island camp site will be removed with the empty barrels and scrap steel to Arviat or Rankin Inlet by snow cat during the winter of 2018-19.  It is planned for ERM Rescan to complete a sample survey of the site after this removal with the results submitted to the KIA for review.  

E) Baseline Studies

Rescan Environmental Services, on behalf of Starfield, conducted extensive baseline monitoring studies in 2007 of the Ferguson Lake area. These studies were continued and expanded upon in 2008, and included:

• Meteorology

• Hydrology

• Metal Leaching/Acid Rock Drainage (ML/ARD)

• Aquatic Resources and Water Quality

• Fisheries Community and Habitat

• Soils

• Vegetation

• Wildlife

• Traditional Knowledge

• Socio‐economics

Due to budgetary constraints, no additional Environmental Baseline Studies have been undertaken.  During the 2014 exploration program, ERM Rescan will continue to gather weather data, and collect and analyze additional water sample as needed on behalf of CNRD.  Data will continue to be incorporated in the project’s baseline environmental studies.  CNRD is aware the Property is located within a major caribou migration route active May 1–July 31, and will not conduct any activity in the area in 2018 until after this period such as to minimize any disturbances. 
F) Removal of Scrap Metal, Storage Batteries and Used Drums

During the 2011 Exploration Program, the driller’s laydown yard was cleaned of debris, and scrap metal related to previous drilling and stockpiled for future transport. Additionally, old storage batteries were transported and removed from the site.  As previously mentioned and part of the 2018-19 Work Program, it is planned to ship the scrap steel, material from the 2012 clean-up plus about 3,000 crushed and bundled empty fuel drums from the project.  Removed waste material will not be dumped on Nunavut territory but shipped from Arviat or Rankin Inlet to be disposed at an accredited dump site.  

G)  Socio-Economic Impacts and Benefits

During the 2008 - 2012 exploration programs, Starfield hired locally whenever possible with CNRD continuing this practice in 2013-15. During this period, northern businesses were also utilized whenever possible and practicable.  In 2018, Local people will be hired whenever possible and CNRD will continue to utilize the services of northern businesses, as much as is practicable.
H)  Archaeological, Traditional Knowledge and Socio‐Economic Studies

As time and budgets permit, this work will continue with the KIA to identify the repository for the Traditional Knowledge studies so that they are secure and also accessible to the communities.  
Starfield Resources had since 2008 undertaken a modest donations program for communities in the Kivalliq Region, as well as continue being a member of the Kivalliq Chamber of Commerce and the NWT & NU Chamber of Mines.  It had been important to Starfield and now CNRD to not overstate the project, and to “manage” community and government expectations of the project and the possible impacts and benefits to the communities.
CONCLUSIONS

In summary, since the transfer of the Ferguson Lake project to CNRD, due to the rising costs of development and decreasing metal prices, especially for Ni, the present development plan remains on hold and the status of the property has been downgraded to the level of an exploration venture.   The property is being re-assessed as an exploration venture with the purpose of increasing the material value of the project by defining new higher grade resource bodies, improving the economic efficiency of the overall metallurgical recovery, and including the value of the platinum group metals in any economic analysis.  Moving forward through 2018, this policy will continue under this management pending a significant advancement of the project towards economic feasibility.
Prepared by Trevor Boyd, Project Manager
Canadian North Resources and Development Corp.

June 8, 2018
8

