
[image: image1]
“Job Safety Analysis”
JSA # 6002

	Title of Job/Operation:
	Operating on Terrain
	JSA Created on: 
	February 21, 2002

	Person(s) performing Job:
	Driller & Helpers
	Employee(s) Observed:
	

	Dept.: 
	Drilling Services
	Analysis Done by:
	KL

	Dept. Supervisor
	
	Analysis approved by:
	KL

	PPE Required:
	Regular Surface PPE
	Notes:
	6


	Sequence of Basic Job Steps
	Potential Hazards, Unsafe Acts or Conditions
	Recommended Safe Job Procedure 

	1. Inspect & examine terrain for drill equipment travel and operation
	(FS, OE, E) Slip, trip, fall. Possible exposure to contamination, poison oak and ivy. Possible ditches, soft ground ruts, rocks, stumps, ice, snow, water, mud.  Overhead or underground utilities, branches or tree limbs.
	1. Use caution when examining the terrain and the site.  Mark any hazards that may be in the way.  Pick a stable and clear route to drill site.  Level off terrain or build roads to the drill location.  Clear area or build pad at drill location if needed.

	2. Transport equipment to drill location
	(CW, CB, SB) Obstructions, exposure to uneven ground, ditches, stumps, debris, ruts, rocks, ice, snow, trees and limbs.  Tipping, rollover.  Getting stuck or losing control while moving.
	2. Use caution when moving on uneven or soft terrain.   Use spotters while moving forward or back.  Use mud or stabilization mats if needed.

	3. Set up equipment
	(FS FB, OE, E) Tipping or roll over of equipment, contact with existing branches or trees.
	3. Wear proper PPE.  Use pads under stabilizers.  Level equipment.  Stay clear of overhead and underground utilities or tree branches or limbs.  Be alert when setting up equipment and raising the tower.  Mark worksite including the exclusion zone.


	1.
Struck By (SB)
	5.
Caught On (CO)
	9.
Fall to Below (FB)

	2.
Struck Against (SA)
	6.
Caught In (CI)
	10.
Overexertion (OE)

	3.
Contacted By (CB)
	7.
Caught Between (CBT)
	11.
Exposure (E)

	4.
Contact With (CW)
	8.
Fall - Same Level (FS)
	


Page 1 of 1

Last Modified: 2008/01/17

[image: image1][image: image2.wmf]