
[image: image1]
“Job Safety Analysis”
JSA # 6010

	Title of Job/Operation:
	To Prepare the Drill for Geophysics or Gyro Survey
	JSA Created on: 
	November 10, 2005

	Person(s) performing Job:
	Drillers and helpers
	Employee(s) Observed:
	

	Dept.: 
	Surface Diamond Drilling
	Analysis Done by:
	Ron Bourgeois

	Dept. Supervisor
	
	Analysis approved by:
	

	PPE Required:
	Regular Surface PPE; fall arrest system
	Risk Level:
	2


	Sequence of Basic Job Steps
	Potential Hazards, Unsafe Acts or Conditions
	Recommended Safe Job Procedure 

	1. Prepare the work area for the survey.
	
	1. When instructions are given to prepare the drill for a Geophysics or Gyro survey the Drill Runner will be advised at that time if the rods are to be pulled or left in the hole.

2. If the rods are pulled it should be made aware at this time if they are to be permanently stored in the sloop or placed out of the way in the drill shack.

	2. Determine if a Boart Longyear employee will be present during the survey.
	
	3. In the case of a Gyro survey when the contractor is using his own equipment, this lockout procedure will be applied, otherwise a Boart Longyear Driller Runner will be present to operate the drill and lockout procedure will not be required.

	3. Prepare the lockout for the drill.
	
	4. The drill will be left in pulling mode.

5. The running block will be raised up and left at a fare good distance (20 to 30 feet; 6 to 9 m) from the floor.

	4. 
	(SB) Struck by running block if lever is not properly engaged.

(SB) Struck by running block if lock out procedure is not used and it is accidentally disengaged.

 

(CI) Caught in moving equipment if drill is powered up accidentally.
	6. The brake lever will be applied so that the brake is engaged.

7. The brake lever will be secured in place with a chain and lock so it can not be engaged either accidentally or intentionally.

8. The lock will be accompanied with a "Do Not Operate" tag.

9. The electric starter switch will be locked out and tagged out.

10. The tag will indicate "Shut down for Geophysics survey" or something to that effect.

	5. Leave area clean and ready for visitors.
	(E; FS) Expose to slips and falls if the floors are greasy and Fall same level if there are tripping hazards left on the floor.
	11. The drill floor will be washed and the drill will be left clean and tidy.

12. In winter conditions, heat will be supplied as requested.


	1.
Struck By (SB)
	5.
Caught On (CO)
	9.
Fall to Below (FB)

	2.
Struck Against (SA)
	6.
Caught In (CI)
	10.
Overexertion (OE)

	3.
Contacted By (CB)
	7.
Caught Between (CBT)
	11.
Exposure (E)

	4.
Contact With (CW)
	8.
Fall - Same Level (FS)
	


Page 1 of 1

Last Modified: 2008/01/17


[image: image1][image: image2.wmf]