### Spill Mitigation

- All equipment with internal combustion engines shall have secondary containment – placed in berm when possible, drip pans/spill pads used when not possible.

- All POL storage containers will be placed in secondary containment.

- All secondary containment will be inspected daily at a minimum and drained as required. All drainage water will be collected and disposed of in accordance with (IAW) the Canadian Environmental Protection Act.

- Product Transfer Area consisting of drip pan and spill pad will be used at minimum. Fuel spigots will be used to transfer product from storage container into equipment.

- French drain will be dug around the perimeter of the site, with a hydrophobic boom placed at the outfall of the drain, to catch any POL in any runoff.

- Spill kits will be placed in readily available areas for operators to use.

- All operators will be briefed by Water, Fuels, and Environmental Technicians (WFEs) in regards to (IRT) the spill response plan and equipment prior to deployment. The Spill Response Plan (SRP) will be available on site.

### Spill Response

- Upon discovery of a spill, DND personnel will immediately:

  - Isolate the source of the leak and stop the source if safe to do so;

  - Shut off any affected equipment; and

  - Notify COC of the spill and ask for assistance as required.

- Any contaminated soil will be dug up and disposed of IAW the Canadian Environmental Protection Act. Remediation will continue until satisfactory visual and olfactory inspection is complete by WFEs.

- All consumables used during the remediation process will be disposed of accordingly.

- Any spill, regardless of size, will be reported via a spill report (see Annex A) through the COC to municipal authorities and the Nunavut Department of Environment.

- Civilian authorities will receive spill response report within 24 hours of any event.

Spill Report

Time:

Date:

Location:

Name of person reporting spill:

Contact information:

Name:

Phone:

Volume of spilled product:

Name of spilled product:

Source of spilled product:

Brief description of how spill occurred:

Did spill reach waterway? Y / N

Reported By:

Signature:

Date: 						Name:

Unit Environmental Officer

Signature:

Date: 						Name: