Municipality of Clyde River Water Licence No. 3BM-CLY1924 Water Licence Renewal and Amendment Application

EXECUTIVE SUMMARY

The Municipality of Clyde River is submitting to the Nunavut Water Board (NWB) a Water Licence Renewal Application for Water Licence No. 3BM-CLY1924.  The Hamlet is also seeking a twenty (20) year renewal of the Licence which would be valid until 2044. There are no changes in operations and no changes to the licenced facilities.

The Municipality of Clyde River utilizes Water Source Lake (also known as Emirtavik Lake) for potable water with the authorized quantity of water not to exceed 53,600 cubic metres annually or 299 cubic metres daily. Water is pumped from Water Source Lake throughout the year into the adjacent water treatment plant for municipal purposes. Based on the estimated projection of 1,755 by 2044, an increase to the annual water use withdrawal limit from Water Source Lake from 53,600 cubic metres to 72,000 cubic metres is requested. No changes to the daily water use withdrawal limit is required.

Sewage is disposed of and treated in a two-cell storage lagoon with the capacity of a period of one year. During the open water season, the sewage is discharged by pump into a natural tundra vegetated filter strip wetland area 23.5 hectares in size (720 metres long by 325 metres wide) for additional treatment prior to release into the environment at Patricia Bay.

The solid waste disposal facility consists of a fenced disposal area for municipal solid waste (MSW), a separate bulk metal disposal area, and a separate hazardous materials area.

There are no changes to the infrastructure or operations of the Sewage Disposal and Solid Waste Disposal facilities.

The Municipality of Clyde River is also requesting amendments to various items of the existing water licence outlined in the document with the submission titled Water Licence 3BM-CLY1924 Renewal and Amendment Application: Amendment Justification Summary