| Licence Condition |  | Status of Compliance | Short Term Plan for Compliance | Schedule for Implementation of Short Term Plan for Compliance (including funding status where applicable to municipal developments) | Long Term Plan for Compliance | Schedule for Implementation of Long Term Plan for Compliance (including funding status where applicable to municipal developments) |
| --- | --- | --- | --- | --- | --- | --- |
| Part B | General Conditions |  |  |  |  |  |
| B-1 | The Licensee shall file an Annual Report with the Board not later than March 31st | Non-Compliance | It is CGS’s intention to ensure the communities submit the plans from 2004-2010 with all available information, operating from the most recent plan, i.e. 2010 and working backwards to the oldest plan, i.e. 2004 | - It is CGS’s intention to ensure that the annual report for 2012 be submitted on before March 31st, 2013. - Annual reports for prior years will be submitted prior to March 31st, 2014. - The community visit by exp is intended to be held prior to the end of February 2013 with a subsequent report issued prior to March 31st, 2013 | GN will monitor of ensure Annual Report submission by March 31st. This monitoring will include weekly checks of the Hamlet’s log sheets, along with monthly water committee meeting to discuss and action any issues. | All outstanding Annual Reports submitted on or before March 31st, 2013 |
| B-2 | The Licensee shall comply with the “Monitoring Program” described in this Licence, and any amendments to the “Monitoring Program” as may be made from time to time, pursuant to the conditions of this Licence. | Non-compliance - Please refer to section H for specific compliance issues | CGS has retained the services of exp to assist in the development and implementation of the compliance plan. Exp’s mandate will include: - preparation of a QA/QC plan which identifies sampling protocols and testing requirements - develop a monitoring action plan for the community which meets the requirements of the water licence | - Action Plan including procedures and QA/QC protocols prepared by June 1st, 2013 - Recommendations regarding training program submitted to CGS by March 31st, 2013 - Training program to be started summer 2013 | Develop a training plan for a TRAIN THE TRAINER to be delivered in 2013/14 for GN staff All reports to be held in a central data base accessible to GN staff for review and action | All samples completed in accordance with the water licence and submitted on or before March 2014 for the 2013 season |
|  |  |  | - recommendations with regards to a new training program for the monitoring program will be made as part of the community visit report |  |  |  |

| Licence Condition |  | Status of Compliance | Short Term Plan for Compliance | Schedule for Implementation of Short Term Plan for Compliance (including funding status where applicable to municipal developments) | Long Term Plan for Compliance | Schedule for Implementation of Long Term Plan for Compliance (including funding status where applicable to municipal developments) |
| --- | --- | --- | --- | --- | --- | --- |
| B-5 | The Licensee shall, within ninety (90) days after the first visit by the Inspector following issuance of this Licence, post the necessary signs to identify the stations of the “Monitoring Program”. All signage postings shall be in the Official Languages of Nunavut. | Non compliance as per 2011 inspection reports, signed, sent off and posted | As part of the community visit, exp, will visit each of the monitoring points and verify presence or absence of signage. As part of exp’s mandate they will prepare a tender document for procurement of the required signs with installation by Hamlet staff | - The site visit report by exp which will identify deficiencies and signage for the monitoring sites will be issued prior to March 31st, 2013. - Procurement of signage prior to June 30th, 2013 - Signs will be installed prior to August 31st, 2013 | Monitoring Stations will be placed on the inspection checklist for the MTO (Municipal Technical Officer) to be indentified if this reverts back to non-compliance INSPECTION REPORT FOR REGULAR INSPECTIONS Inspection report enclosed | Regular inspections by GN staff during community visits |
| B-9 | The Licensee shall submit one paper copy and one electronic copy of all reports, studies, and plans to the Board. Reports or studies submitted to the Board by the Licensee shall include a detailed executive summary in Inuktitut. | Non compliance | exp will review with the NWB to identify the existing outstanding items and submissions | Community site visit will report status of outstanding reports, and will be issued by March 31, 2013, Any available reports studies, or plans to be submitted prior to June 1st, 2013 | CGS will request additional funding to ensure communities have the resources to complete reports and studies in both languages. | The Hamlet of Gjoa Haven will have these translated before March 31st 2014. |
| B-11 | The Licensee shall submit to the Board for approval, within ninety (90) days of Licence issuance or upon the filing of any application in relation to the Licence within that time, a Plan for Compliance that clearly demonstrates the measures the Licensee will undertake, including an implementation schedule, to achieve full compliance with the conditions of this Licence, including the issues raised in the Inspector’s Reports. | Non-compliance | - CGS has retained the services of exp to assist with the preparation of a compliance plan. - Draft Compliance Plan will be reviewed with AANDC during preparation | Compliance plan will be submitted prior to November 2, 2012. | - Annual review of need for preparation of a new Compliance Plan. - Prepare Compliance Plans as required | On or before March 31st of each year. |

| Licence Condition |  | Status of Compliance | Short Term Plan for Compliance | Schedule for Implementation of Short Term Plan for Compliance (including funding status where applicable to municipal developments) | Long Term Plan for Compliance | Schedule for Implementation of Long Term Plan for Compliance (including funding status where applicable to municipal developments) |
| --- | --- | --- | --- | --- | --- | --- |
| Part D | Conditions Applying To Waste Disposal |  |  |  |  |  |
| D-2 | All Effluent discharged from the Sewage Disposal Facility at Monitoring Program Station KIM-3 and the Enhanced Sewage Disposal Facility at Monitoring Program Stations KIM-6 and KIM-7 shall not exceed the following Effluent quality limits: | Non compliance as per 2011 inspection | - Community site visit report will be reissued by March 31st , 2013. - Recommendations with regards to a new training program for the monitoring program will be made as part of the community visit report - The monitoring program shall be provided to the Hamlet June 1st, 2013 | - Arrangement to be made with a Canadian Association of Environmental Analytical Laboratories (CAEAL) Certified Laboratory to perform the analysis and report back to the community - Train municipal staff in sampling by June 1st 2013 | Ongoing training for the monitoring program Develop a training plan for a TRAIN THE TRAINER to be delivered in 2013/14 for GN staff | All samples completed in accordance with the water licence and submitted on or before March 2014 for the 2013 season |
| D-7 | The Licensee shall segregate and store all hazardous materials and/or hazardous waste, including waste oil, within the Solid Waste Disposal Facility in a manner as to prevent the deposit of deleterious substances into any water until such a time as proper disposal arrangements are made. | Non compliance | As part of the exp community visit assessment of the solid waste facility and current practices will be undertaken. Following the community site visit a report outlining the findings and making recommendations with regards to compliance issues will be prepared and issued | - Community site visit February 28th, 2013 - Community site visit report March 31st, 2013 - Action the operational recommendations August 31,2013 | Action plan for the non-operational recommendations Develop new MSW strategy | The development of the new Solid Waste Strategy and the Capital Action Plan July 31st 2013. |
| Part F | Conditions Applying To Operation And Maintenance |  |  |  |  |  |
| F-2 | The Licensee shall submit to the Board for approval, immediately following the first operating season of the Enhanced Sewage Disposal Facility, a revision to the Operations and Maintenance (O&M) Manual referred to in Part F Item 1, The Manual shall take into consideration the comments received during the application review process. | In compliance; awaiting commissioning | Status of O&M Plans and Manuals to be reviewed during site visit. - Sewage O&M - Solid Waste O&M - Hazardous waste - Spills - Water course modifications - Monitoring QA/QC | - Report status of O&M Manual prior to March 15, 2013. - Prepare O&M Manual as required and submit prior to July 1st, 2013. | Annual review of status and revise if required. | Amend if required prior to March 31st submission of Annual Report |

| Licence Condition |  | Status of Compliance | Short Term Plan for Compliance | Schedule for Implementation of Short Term Plan for Compliance (including funding status where applicable to municipal developments) | Long Term Plan for Compliance | Schedule for Implementation of Long Term Plan for Compliance (including funding status where applicable to municipal developments) |
| --- | --- | --- | --- | --- | --- | --- |
| F-2 cont’d |  |  | Prepare required plans and manuals - Status of O&M Manual (Water Supply) to be reviewed during site visit. Report subsequent to visit. - Prepare O&M Manuals as required based on site visit findings |  |  |  |
| Part G | Conditions Applying to Abandonment and Restoration |  |  |  |  |  |
| G-1 | The Licensee shall submit to the Board for approval an Abandonment and Restoration Plan, at least six (6) months prior to a) abandoning of any facilities and b) the construction of new facilities to replace existing ones. | Non-Compliance | - CGS has retained the services of exp to assist with the preparation of A&R Plan. | - Report on scope of plan prior to March 1, 2013. - Follow up visit to gather technical information and indentify all potential sites August 1, 2013. - Report on follow-up visit by September 15, 2013. | Annual review of need for A&R Plans, and revise if required including schedule for preparation | - Submit A&R all outstanding Plans March 31st 2015 - Amend if required prior to March 31st submission of Annual Report. |