
ANNUAL REPORT 






FOR THE HAMLET OF PANGNIRTUNG, 2012


YEAR BEING REPORTED:  2012


The following information is compiled pursuant to the requirements of Part B, Item 1 of Water License # NWB 3- PAN 0810 issued to the Hamlet of Pangnirtung.
i) - iii)
tabular summaries of all data generated under the “Monitoring Program”; monthly and annual quantities in cubic metres of freshwater obtained from all sources; monthly and annual quantities in cubic metres of each and all wastes discharged;

Attached are quantities of water used as reported in our On Tap Water Delivery System and the estimated discharge of sewage waste based on quantities used.
	Month Reported
	Quantity of Water Obtained from all sources (litres)
	Quantity of Sewage Waste Discharged
(Estimated)

	January 2012
	4,367,212.60
	Same

	February 2012
	4,430,045.90
	Same

	March 2012
	4,732,710.10
	Same

	April 2012
	4,226,844.20
	Same

	May 2012
	4,480,016.50
	Same

	June 2012
	4,267,176.00
	Same

	July 2012
	4,371,988.50
	Same

	August 2012
	4,345,826.90
	Same

	September 2012
	3,986,482.20
	Same

	October 2012
	4,551,534.10
	Same

	November 2012
	4,099,388.80
	Same

	December 2012
	4,344,604.50
	Same

	ANNUAL TOTAL
	52,203,830.30
	52,203,830.30


             Note: The monthly water consumption volume is considered equal to the discharge
               volume because there is no meter at the end of the discharge pipe.

iv. a summary of modifications and/or major maintenance work carried out on the Water Supply and Waste Disposal Facilities, including all associated structures and facilities;


Design for optimization of the WWTP is in progress by exp. Services Inc. Design for the Water Truck Fill station is almost completed by Arktis Solutions Inc. Also design for the waste management process is in advance stage by exp. Services Inc.

v. a list of unauthorized discharges and summary of follow-up action taken;    


      Nil

vi. a summary of any abandonment and restoration work completed during the year and an outline of any work anticipated for the next year; 


Once the WWTP will be optimized and new waste management facility is built and commissioned, the existing ditch so called natural lagoon and the existing land fill site will be abandoned and restored. All liquid wastes of the Community are expected to be treated by the WWTP only once optimization is completed.
           ________________________________________________________________________ 
vii. a summary of any studies requested by the Board that relate to waste disposal, water use or reclamation, and a brief description of any future studies planned;

______________________________________________________________________

                  The studies on solid waste management was completed by Arktis Solutions Inc. 

                  A new guidelines has been developed. The Consultant exp. Services Inc. is designing 

                  a new facility satisfying the conditions of the new guidelines.     
           ___________________________________________________________________           

viii. any other details on water use or waste disposal requested by the Board by November 1st of the year being reported; and





Wastes management in the Community are the major concerns of the Nunavut Water Board. 
           _________________________________________________________________

ix. updates or revisions to the approved Operation and Maintenance Plans.


                 O&M manual of the Water Truck fill station will be updated once the new facility is 
                 built and commissioned. The O&M manual of the WWTP will be updated accordingly 
                 once the optimization of the existing facility is completed. Also a new O&M manual 
                 for the Waste management will be made available once the new facility is built and 
                 commissioned.
                              _________________________________________________________________

                 ADDITIONAL INFORMATION THAT THE LICENSEE DEEMS USEFUL:

                     Improvement of all the Environmental facilities under water License for compliancy.

                    FOLLOW-UP REGARDING INSPECTION/COMPLIANCE CONCERNS:


                    Hamlet is working with CGS for AANDC inspection report and Water License 
                Compliancy. Hamlet has engaged in extended sampling program for water, wastewater 

                and the leachate coming from the landfill site.

[image: image1.emf]B12-31429_R.pdf



 EMBED AcroExch.Document.7  [image: image2.emf]B12-23350_R.pdf



 EMBED AcroExch.Document.7  [image: image3.emf]B12-18211_R.pdf



 EMBED AcroExch.Document.7  [image: image4.emf]B12-09434_R.pdf


Page 1 of 3

_1420872648.pdf


WWTP


20-Sep-12DATE REPORTED:


Caduceon Environmental Laboratories


613-526-1244


2378 Holly Lane 
Ottawa Ontario K1V 7P1


613-526-0123Tel:
Fax:


JOB/PROJECT NO.:


Final Report
REPORT No. B12-23350


Hamlet of Pangnirtung
P.O. Box 253, 
Pangnirtung Nunavut X0A 0R0 Canada


Report To:


Attention: Billy Qaqasiq


12-Sep-12DATE RECEIVED:


P.O. NUMBER:


WATERWORKS NO.Waste WaterSAMPLE MATRIX:


C.O.C.: ---


CERTIFICATE OF ANALYSIS


Parameter Units M.D.L.
Reference 


Method
Date/Site 
Analyzed


Effluent Effluent - 
Dump Site


Client I.D.


B12-23350-1 B12-23350-2Sample I.D.
12-Sep-12 12-Sep-12Date Collected


BOD 66 1020mg/L 3 SM 5210B 14-Sep-12/O
Hardness (as CaCO3) 31mg/L 1 SM 3120 14-Sep-12/O
Alkalinity (as CaCO3) 133mg/L 5 SM 2320B 12-Sep-12/O
Conductivity 757µmho/cm 1 SM 2510B 13-Sep-12/O
pH 7.29pH Units SM 4500H 12-Sep-12/O
Total Suspended Solids 156 3580mg/L 3 SM 2540D 14-Sep-12/O
Nitrite (N) 19.5mg/L 0.1 SM4110C 14-Sep-12/O
Nitrate (N) 0.5mg/L 0.1 SM4110C 14-Sep-12/O
Chloride 56.2mg/L 0.5 SM4110C 14-Sep-12/O
Sulphate 35mg/L 1 SM4110C 14-Sep-12/O
Ammonia (N)-Total 35.4mg/L 0.01 MOEE 3364 13-Sep-12/O
Phenolics < 0.001mg/L 0.001 MOEE 3179 13-Sep-12/O
Total Organic Carbon 19.3mg/L 0.2 EPA 415.1 17-Sep-12/O
Aluminum (total) 1.72mg/L 0.01 SM 3120 14-Sep-12/O
Arsenic 0.0360mg/L 0.0005 EPA 200.8 17-Sep-12/O
Calcium 7.93mg/L 0.02 SM 3120 14-Sep-12/O
Cadmium < 0.005mg/L 0.005 SM 3120 14-Sep-12/O
Chromium 0.003mg/L 0.002 SM 3120 14-Sep-12/O
Cobalt < 0.005mg/L 0.005 SM 3120 14-Sep-12/O
Copper 0.138mg/L 0.002 SM 3120 14-Sep-12/O
Iron (Total) 1.07mg/L 0.005 SM 3120 14-Sep-12/O
Lead < 0.02mg/L 0.02 SM 3120 14-Sep-12/O
Mercury 0.00002mg/L 0.00002 SM 3112 B 13-Sep-12/O
Magnesium 2.62mg/L 0.01 SM 3120 14-Sep-12/O
Manganese (Total) 0.036mg/L 0.001 SM 3120 14-Sep-12/O
Nickel < 0.01mg/L 0.01 SM 3120 14-Sep-12/O
Potassium 22.3mg/L 0.1 SM 3120 14-Sep-12/O


Page 1 of 2.


Gord Murphy 
Lab Supervisor


���������	
������
��	
������	�������
��������	��	���
�����
��
�����
��������������	
������	�

������	
���������	�
����������
�����	�

�����
�
	����
	���	���
������
��	������
� ��������� ��
������	���� �����	��
�
�


M.D.L. = Method Detection Limit
Lab Supervisor


Gord Murphy 


Site Analyzed=K-Kingston,W-Windsor,O-Ottawa,R-Richmond Hill







WWTP


20-Sep-12DATE REPORTED:


Caduceon Environmental Laboratories


613-526-1244


2378 Holly Lane 
Ottawa Ontario K1V 7P1


613-526-0123Tel:
Fax:


JOB/PROJECT NO.:


Final Report
REPORT No. B12-23350


Hamlet of Pangnirtung
P.O. Box 253, 
Pangnirtung Nunavut X0A 0R0 Canada


Report To:


Attention: Billy Qaqasiq


12-Sep-12DATE RECEIVED:


P.O. NUMBER:


WATERWORKS NO.Waste WaterSAMPLE MATRIX:


C.O.C.: ---


CERTIFICATE OF ANALYSIS


Parameter Units M.D.L.
Reference 


Method
Date/Site 
Analyzed


Effluent Effluent - 
Dump Site


Client I.D.


B12-23350-1 B12-23350-2Sample I.D.
12-Sep-12 12-Sep-12Date Collected


Sodium 65.7mg/L 0.2 SM 3120 14-Sep-12/O
Zinc 0.178mg/L 0.005 SM 3120 14-Sep-12/O
Fecal Coliform 188000cfu/100mL 1 MOE E3371 13-Sep-12/O
Oil & Grease-Total < 1.0mg/L 1.0 SM 5520 19-Sep-12/K


Page 2 of 2.


Gord Murphy 
Lab Supervisor


���������	
������
��	
������	�������
��������	��	���
�����
��
�����
��������������	
������	�

������	
���������	�
����������
�����	�

�����
�
	����
	���	���
������
��	������
� ��������� ��
������	���� �����	��
�
�


M.D.L. = Method Detection Limit
Lab Supervisor


Gord Murphy 


Site Analyzed=K-Kingston,W-Windsor,O-Ottawa,R-Richmond Hill






_1420872649.pdf


WWTP


21-Dec-12DATE REPORTED:


Caduceon Environmental Laboratories


613-526-1244


2378 Holly Lane 
Ottawa Ontario K1V 7P1


613-526-0123Tel:
Fax:


JOB/PROJECT NO.:


Final Report
REPORT No. B12-31429


Hamlet of Pangnirtung
P.O. Box 253, 
Pangnirtung Nunavut X0A 0R0 Canada


Report To:


Attention: Billy Qaqasiq


12-Dec-12DATE RECEIVED:


P.O. NUMBER:


WATERWORKS NO.Waste WaterSAMPLE MATRIX:


C.O.C.: G27374


CERTIFICATE OF ANALYSIS


Parameter Units M.D.L.
Reference 


Method
Date/Site 
Analyzed


Influent - 4:00 Influent - 4:05 Effluent - 4:15 Effluent - 4:30Client I.D.
B12-31429-1 B12-31429-2 B12-31429-3 B12-31429-4Sample I.D.
12-Dec-12 12-Dec-12 12-Dec-12 12-Dec-12Date Collected


BOD 632 359 66 57mg/L 3 SM 5210B 14-Dec-12/O
Hardness (as CaCO3) 48 50 50 46mg/L 1 SM 3120 19-Dec-12/O
Alkalinity(CaCO3) to pH4.5 434 433 316 314mg/L 5 SM 2320B 13-Dec-12/O
Conductivity @25°C 1260 1230 1040 1040µmho/cm 1 SM 2510B 13-Dec-12/O
pH @25°C 7.20 7.17 7.70 7.68pH Units SM 4500H 13-Dec-12/O
Total Suspended Solids 176 176 112 92mg/L 3 SM 2540D 14-Dec-12/O
Nitrite (N) 0.2 < 0.1 6.9 6.9mg/L 0.1 SM4110C 13-Dec-12/O
Nitrate (N) 0.2 0.1 0.2 0.2mg/L 0.1 SM4110C 13-Dec-12/O
Chloride 64.4 65.4 63.2 63.2mg/L 0.5 SM4110C 13-Dec-12/O
Sulphate 9 24 34 34mg/L 1 SM4110C 13-Dec-12/O
Ammonia (N)-Total 92.3 89.6 62.3 69.6mg/L 0.01 MOEE 3364 13-Dec-12/O
Phenolics 0.022 0.023 0.006 0.004mg/L 0.001 MOEE 3179 13-Dec-12/O
Total Organic Carbon 119 83.8 42.2 33.5mg/L 0.2 EPA 415.1 14-Dec-12/O
Aluminum (total) 1.72 1.72 1.23 1.16mg/L 0.01 SM 3120 19-Dec-12/O
Arsenic 0.0009 0.0009 0.0007 0.0007mg/L 0.0005 EPA 200.8 18-Dec-12/O
Calcium 13.2 13.7 14.0 12.8mg/L 0.02 SM 3120 19-Dec-12/O
Cadmium < 0.005 < 0.005 < 0.005 < 0.005mg/L 0.005 SM 3120 19-Dec-12/O
Chromium 0.004 0.004 0.004 0.003mg/L 0.002 SM 3120 19-Dec-12/O
Cobalt < 0.005 < 0.005 < 0.005 < 0.005mg/L 0.005 SM 3120 19-Dec-12/O
Copper 0.258 0.257 0.134 0.133mg/L 0.002 SM 3120 19-Dec-12/O
Iron (Total) 1.71 1.72 1.05 1.04mg/L 0.005 SM 3120 19-Dec-12/O
Lead < 0.02 < 0.02 < 0.02 < 0.02mg/L 0.02 SM 3120 19-Dec-12/O
Mercury 0.00014 0.00014 0.00007 0.00009mg/L 0.00002 SM 3112 B 18-Dec-12/O
Magnesium 3.76 3.81 3.52 3.47mg/L 0.01 SM 3120 19-Dec-12/O
Manganese (Total) 0.069 0.068 0.038 0.037mg/L 0.001 SM 3120 19-Dec-12/O
Nickel < 0.01 < 0.01 < 0.01 < 0.01mg/L 0.01 SM 3120 19-Dec-12/O
Potassium 40.8 40.2 28.5 29.1mg/L 0.1 SM 3120 19-Dec-12/O
Sodium 68.0 67.2 61.1 62.2mg/L 0.2 SM 3120 19-Dec-12/O


Page 1 of 2.


Gord Murphy 
Lab Supervisor


���������	
������
��	
������	�������
��������	��	���
�����
��
�����
��������������	
������	�

������	
���������	�
����������
�����	�

�����
�
	����
	���	���
������
��	������
� ��������� ��
������	���� �����	��
�
�


M.D.L. = Method Detection Limit
Lab Supervisor


Gord Murphy 


Site Analyzed=K-Kingston,W-Windsor,O-Ottawa,R-Richmond Hill







WWTP


21-Dec-12DATE REPORTED:


Caduceon Environmental Laboratories


613-526-1244


2378 Holly Lane 
Ottawa Ontario K1V 7P1


613-526-0123Tel:
Fax:


JOB/PROJECT NO.:


Final Report
REPORT No. B12-31429


Hamlet of Pangnirtung
P.O. Box 253, 
Pangnirtung Nunavut X0A 0R0 Canada


Report To:


Attention: Billy Qaqasiq


12-Dec-12DATE RECEIVED:


P.O. NUMBER:


WATERWORKS NO.Waste WaterSAMPLE MATRIX:


C.O.C.: G27374


CERTIFICATE OF ANALYSIS


Parameter Units M.D.L.
Reference 


Method
Date/Site 
Analyzed


Influent - 4:00 Influent - 4:05 Effluent - 4:15 Effluent - 4:30Client I.D.
B12-31429-1 B12-31429-2 B12-31429-3 B12-31429-4Sample I.D.
12-Dec-12 12-Dec-12 12-Dec-12 12-Dec-12Date Collected


Zinc 0.406 0.400 0.201 0.164mg/L 0.005 SM 3120 19-Dec-12/O
Oil & Grease-Total 64.4 104 3.0 2.3mg/L 1.0 SM 5520 21-Dec-12/K
Fecal Coliform 51000000 75000000 480000 510000cfu/100mL 1 MOE E3371 13-Dec-12/O


Page 2 of 2.


Gord Murphy 
Lab Supervisor


���������	
������
��	
������	�������
��������	��	���
�����
��
�����
��������������	
������	�

������	
���������	�
����������
�����	�

�����
�
	����
	���	���
������
��	������
� ��������� ��
������	���� �����	��
�
�


M.D.L. = Method Detection Limit
Lab Supervisor


Gord Murphy 


Site Analyzed=K-Kingston,W-Windsor,O-Ottawa,R-Richmond Hill






_1420872647.pdf


W.W.


27-Jul-12DATE REPORTED:


Caduceon Environmental Laboratories


613-526-1244


2378 Holly Lane 
Ottawa Ontario K1V 7P1


613-526-0123Tel:
Fax:


JOB/PROJECT NO.:


Final Report
REPORT No. B12-18211


Hamlet of Pangnirtung
P.O. Box 253, 
Pangnirtung Nunavut X0A 0R0 Canada


Report To:


Attention: Billy Qaqasiq


20-Jul-12DATE RECEIVED:


P.O. NUMBER:


WATERWORKS NO.Waste WaterSAMPLE MATRIX:


C.O.C.: ---


CERTIFICATE OF ANALYSIS


Parameter Units M.D.L.
Reference 


Method
Date/Site 
Analyzed


EffluentClient I.D.
B12-18211-1Sample I.D.


15-Jul-12Date Collected


BOD 54mg/L 3 SM 5210B 21-Jul-12/O
Hardness (as CaCO3) 35mg/L 1 SM 3120 20-Jul-12/O
Alkalinity (as CaCO3) 147mg/L 5 SM 2320B 20-Jul-12/O
Conductivity 634µmho/cm 1 SM 2510B 20-Jul-12/O
pH 7.34pH Units SM 4500H 20-Jul-12/O
Total Suspended Solids 172mg/L 3 SM 2540D 25-Jul-12/O
Nitrite (N) 2.7mg/L 0.1 SM4110C 20-Jul-12/O
Nitrate (N) 0.2mg/L 0.1 SM4110C 20-Jul-12/O
Chloride 55.8mg/L 0.5 SM4110C 20-Jul-12/O
Sulphate 38mg/L 1 SM4110C 20-Jul-12/O
Ammonia (N)-Total 21.6mg/L 0.01 MOEE 3364 24-Jul-12/O
Phenolics 0.003mg/L 0.001 MOEE 3179 25-Jul-12/O
Total Organic Carbon 24.4mg/L 0.2 EPA 415.1 24-Jul-12/O
Aluminum (total) 1.41mg/L 0.01 SM 3120 20-Jul-12/O
Arsenic 0.0003mg/L 0.0001 EPA 200.8 25-Jul-12/O
Calcium 9.12mg/L 0.02 SM 3120 20-Jul-12/O
Cadmium < 0.005mg/L 0.005 SM 3120 20-Jul-12/O
Chromium 0.002mg/L 0.002 SM 3120 20-Jul-12/O
Cobalt < 0.005mg/L 0.005 SM 3120 20-Jul-12/O
Copper 0.105mg/L 0.002 SM 3120 20-Jul-12/O
Iron (Total) 0.942mg/L 0.005 SM 3120 20-Jul-12/O
Lead < 0.02mg/L 0.02 SM 3120 20-Jul-12/O
Mercury 0.00008mg/L 0.00002 SM 3112 B 27-Jul-12/O
Magnesium 2.97mg/L 0.01 SM 3120 20-Jul-12/O
Manganese (Total) 0.035mg/L 0.001 SM 3120 20-Jul-12/O
Nickel < 0.01mg/L 0.01 SM 3120 20-Jul-12/O
Potassium 23.8mg/L 0.1 SM 3120 20-Jul-12/O
Sodium 71.4mg/L 0.2 SM 3120 20-Jul-12/O


Page 1 of 2.


Gord Murphy 
Lab Supervisor


���������	
������
��	
������	�������
��������	��	���
�����
��
�����
��������������	
������	�

������	
���������	�
����������
�����	�

�����
�
	����
	���	���
������
��	������
� ��������� ��
������	���� �����	��
�
�


M.D.L. = Method Detection Limit
Lab Supervisor


Gord Murphy 


Site Analyzed=K-Kingston,W-Windsor,O-Ottawa,R-Richmond Hill







W.W.


27-Jul-12DATE REPORTED:


Caduceon Environmental Laboratories


613-526-1244


2378 Holly Lane 
Ottawa Ontario K1V 7P1


613-526-0123Tel:
Fax:


JOB/PROJECT NO.:


Final Report
REPORT No. B12-18211


Hamlet of Pangnirtung
P.O. Box 253, 
Pangnirtung Nunavut X0A 0R0 Canada


Report To:


Attention: Billy Qaqasiq


20-Jul-12DATE RECEIVED:


P.O. NUMBER:


WATERWORKS NO.Waste WaterSAMPLE MATRIX:


C.O.C.: ---


CERTIFICATE OF ANALYSIS


Parameter Units M.D.L.
Reference 


Method
Date/Site 
Analyzed


EffluentClient I.D.
B12-18211-1Sample I.D.


15-Jul-12Date Collected


Zinc 0.152mg/L 0.005 SM 3120 20-Jul-12/O
Oil & Grease-Total 1.0mg/L 1.0 SM 5520 25-Jul-12/K
Fecal Coliform > 20000cfu/100mL 1 MOE E3371 20-Jul-12/O


Page 2 of 2.


Gord Murphy 
Lab Supervisor


���������	
������
��	
������	�������
��������	��	���
�����
��
�����
��������������	
������	�

������	
���������	�
����������
�����	�

�����
�
	����
	���	���
������
��	������
� ��������� ��
������	���� �����	��
�
�


M.D.L. = Method Detection Limit
Lab Supervisor


Gord Murphy 


Site Analyzed=K-Kingston,W-Windsor,O-Ottawa,R-Richmond Hill






_1420872646.pdf


WWTP


02-May-12DATE REPORTED:


Caduceon Environmental Laboratories


613-526-1244


2378 Holly Lane 
Ottawa Ontario K1V 7P1


613-526-0123Tel:
Fax:


JOB/PROJECT NO.:


Final Report
REPORT No. B12-09434


Hamlet of Pangnirtung
P.O. Box 253, 
Pangnirtung Nunavut X0A 0R0 Canada


Report To:


Attention: Billy Qaqasiq


25-Apr-12DATE RECEIVED:


P.O. NUMBER:


WATERWORKS NO.Waste WaterSAMPLE MATRIX:


C.O.C.: 106953


CERTIFICATE OF ANALYSIS


Parameter Units M.D.L.
Reference 


Method
Date/Site 
Analyzed


EffluentClient I.D.
B12-09434-1Sample I.D.


23-Apr-12Date Collected


BOD 69mg/L 3 SM 5210B 27-Apr-12/O
Hardness (as CaCO3) 31mg/L 1 SM 3120 30-Apr-12/O
Alkalinity (as CaCO3) 274mg/L 5 SM 2320B 26-Apr-12/O
Conductivity 958µmho/cm 1 SM 2510B 26-Apr-12/O
pH 7.63pH Units SM 4500H 26-Apr-12/O
Total Suspended Solids 96mg/L 3 SM 2540D 27-Apr-12/O
Nitrite (N) 6.1mg/L 0.1 SM4110C 26-Apr-12/O
Nitrate (N) 0.2mg/L 0.1 SM4110C 26-Apr-12/O
Chloride 55.1mg/L 0.5 SM4110C 26-Apr-12/O
Sulphate 45mg/L 1 SM4110C 26-Apr-12/O
Ammonia (N)-Total 74.5mg/L 0.01 MOEE 3364 30-Apr-12/O
Phenolics 0.002mg/L 0.001 MOEE 3179 02-May-12/O
Total Organic Carbon 22.6mg/L 0.2 EPA 415.1 30-Apr-12/O
Aluminum (total) 1.21mg/L 0.01 SM 3120 30-Apr-12/O
Arsenic 0.0011mg/L 0.0005 EPA 200.8 30-Apr-12/O
Calcium 8.52mg/L 0.02 SM 3120 30-Apr-12/O
Cadmium < 0.005mg/L 0.005 SM 3120 30-Apr-12/O
Chromium < 0.002mg/L 0.002 SM 3120 30-Apr-12/O
Cobalt < 0.005mg/L 0.005 SM 3120 30-Apr-12/O
Copper 0.090mg/L 0.002 SM 3120 30-Apr-12/O
Iron (Total) 0.773mg/L 0.005 SM 3120 30-Apr-12/O
Lead < 0.02mg/L 0.02 SM 3120 30-Apr-12/O
Mercury 0.00005mg/L 0.00002 SM 3112 B 27-Apr-12/O
Magnesium 2.25mg/L 0.01 SM 3120 30-Apr-12/O
Manganese (Total) 0.033mg/L 0.001 SM 3120 30-Apr-12/O
Nickel < 0.01mg/L 0.01 SM 3120 30-Apr-12/O
Potassium 24.4mg/L 0.1 SM 3120 30-Apr-12/O
Sodium 75.2mg/L 0.2 SM 3120 30-Apr-12/O


Page 1 of 2.


Gord Murphy 
Lab Supervisor


���������	
������
��	
������	�������
��������	��	���
�����
��
�����
��������������	
������	�

������	
���������	�
����������
�����	�

�����
�
	����
	���	���
������
��	������
� ��������� ��
������	���� �����	��
�
�


M.D.L. = Method Detection Limit
Lab Supervisor


Gord Murphy 


Site Analyzed=K-Kingston,W-Windsor,O-Ottawa,R-Richmond Hill







WWTP


02-May-12DATE REPORTED:


Caduceon Environmental Laboratories


613-526-1244


2378 Holly Lane 
Ottawa Ontario K1V 7P1


613-526-0123Tel:
Fax:


JOB/PROJECT NO.:


Final Report
REPORT No. B12-09434


Hamlet of Pangnirtung
P.O. Box 253, 
Pangnirtung Nunavut X0A 0R0 Canada


Report To:


Attention: Billy Qaqasiq


25-Apr-12DATE RECEIVED:


P.O. NUMBER:


WATERWORKS NO.Waste WaterSAMPLE MATRIX:


C.O.C.: 106953


CERTIFICATE OF ANALYSIS


Parameter Units M.D.L.
Reference 


Method
Date/Site 
Analyzed


EffluentClient I.D.
B12-09434-1Sample I.D.


23-Apr-12Date Collected


Zinc 0.128mg/L 0.005 SM 3120 30-Apr-12/O
Oil & Grease-Total < 1.0mg/L 1.0 SM 5520 01-May-12/K
Fecal Coliform > 200000cfu/100mL 1 MOE E3371 26-Apr-12/O


Page 2 of 2.


Gord Murphy 
Lab Supervisor


���������	
������
��	
������	�������
��������	��	���
�����
��
�����
��������������	
������	�

������	
���������	�
����������
�����	�

�����
�
	����
	���	���
������
��	������
� ��������� ��
������	���� �����	��
�
�


M.D.L. = Method Detection Limit
Lab Supervisor


Gord Murphy 


Site Analyzed=K-Kingston,W-Windsor,O-Ottawa,R-Richmond Hill






