Appendix G

Description of Arctic Watch Lodge

To Accompany Lodge Aerial Photograph

This photograph shows Arctic Watch Lodge in the center of the image with the Cunningham River at the bottom of the image. The camp faces the river in a northerly direction. The camp has 16 fabric cabins each measuring 12’ x 12’ for accommodation. In the lower portion of the image are two other 12’ x12’ fabric cabins, one is a pantry, the other staff accommodation. The other two out-buildings are a workshop and a storage building. The main building has a “Great Room” 30’ x 70’ this serves as a living room. Two 12’ wide corridors come out from the Great Room. The left hand one, serves as equipment storage (kayak gear, life jackets, etc) it connects to the round section (30’dia) which is a shower house. The other corridor goes to the dining room (30’ dia.). The Kitchen is the corridor coming from the dining room and projecting toward the bottom of the photograph.

The sewage lagoon ( 1.5 m  x  4 m) is at the top right hand side of the image. The “ladder” on the side of the river bank is water intake. The grey water sump is mid-way on the river bank, located on the right hand side of the image in the middle. The river flows from left to right in the image. The current is significant.

Fuel drums are stored on the left hand side of the image. There is one 300-gallon fuel tank (grey in colour) in the center courtyard of the lodge. The remaining square white box in the courtyard is the refrigerator. 

In 2010, we planning to build a separate building (materials are already in place) to house and separate / isolate the generators and any potential spills. 

