[image: image1.png]NASITTUQ



Nunavut Water Board Annual Report 2007

 Submitted by Nasittuq Corporation [image: image3.png]





[image: image6.emf]
ANNUAL REPORT 2007

Submitted to: 

Nunavut Water Board 

For 

Licence:  NWB6FOH0409 - Type “B”

                  FOX-MAIN, Hall Beach, NU

Submitted by:  Nasittuq Corporation

31 March 2008
This annual report is submitted to the Nunavut Water Board (NWB) as stipulated in Para. 1, Part B, General Conditions of the following licence:

Licence: NWB6FOH0409 Type “B”

NUNAVUT WATER LICENCE:  NWB6FOH0409 Type “B”

North Warning System Long Range Radar – FOX-MAIN, Hall Beach, Nunavut
FOX-MAIN, Hall Beach, is a fully attended site, and supports 9 other unattended radar stations within its Zone.  In addition to supporting these sites, work at FOX-MAIN itself includes quarterly preventive/corrective maintenance, in-house regulatory compliance audits and inspections, fuel resupply, and third party related projects.
Water Use 

The normal compliment of personnel at FOX-MAIN is 15 people.  Among these, some members are day employees from the Hall Beach community, and consequently contribute only minimally to the total water consumption at the site.  The calculated total water usage for the site is 821.3 cubic meters.  This is based on 150 litres per person per day, with an average daily occupancy rate of 15 persons.  This gives an average daily usage of 2.3 cubic meters, below the 10 cubic meters daily allowed by the license.  Note that a water meter has since been installed at the site, and for the 2009 report, we will be able to report actual usage.

Hazardous Waste Disposal
The waste management was consistent with the Nasittuq Environmental Protection Plan, meeting the terms and conditions stipulated in the NWB water licence.

The following items were retrograded to Quebec for disposal in 2007.
Disposal facility: Recyclex, 8381, Place Marien, Montreal-EST Quebec. H1B 5W6
Summary of Waste Retrograde in 2007
	LSS-F
	Batteries dry non reg
	1 crate

	LSS-F
	Aircraft grease
	1 crate

	LSS-F
	Waste fuel mixture
	4 drums

	LSS-F
	Waste Varsol
	4 drums

	LSS-F
	Waste fuel filters
	1 drum

	LSS-F
	Waste paint
	11 drums

	LSS-F
	Waste batteries, wet, lead acid
	2 crates

	LSS-F
	Flammable liquid n.o.s
	1 drum

	LSS-F
	Asphalt sealer, non reg
	1 drum

	LSS-F
	Carbon filters
	6 drums

	LSS-F
	Waste oil filters
	2 drums

	LSS-F
	Waste oil
	30 drums


Non Hazardous Solid Waste Disposal

All domestic solid waste was disposed of by burning in a burn bin and buried at a designated on-site landfill.

Refer to Annex A for designated landfill location, with GPS coordinates at FOX-M.
Sewage Discharge

The sewage and greywater at FOX-MAIN, Hall Beach, Nunavut is handled by an advanced wastewater treatment system, Cycle-let®.  The functionality of the system is monitored on-line and maintained by CWC Wastewater Services Inc. on a quarterly basis.  Some of the treated water is recycled as on site urinal/toilet flush water, with the excess passing to an outfall.  Sampled water quality of the treated water is in Annex C.
Refer to Annex B for sewage outfall location, with GPS coordinates at FOX-M.
Monitoring Program

Refer to Annex C for sewage analysis for FOX-M.

Product Spills

Two spills occurred during this reporting period

	Date
	Product
	Quantity
	Cause 
	Locality

	14 Aug 2007
	Glycol
	40 litres
	Coolant hose failure
	Inside LSS-F powerhouse building

	01 Nov 2007
	Glycol
	30 litres
	Engine hose failure
	Inside LSS-F powerhouse building


Revisions to the ERP 

No revisions have been effected during this reporting period.

Progressive reclamation work undertaken 

There was some reclamation at FOX-MAIN in 2007.  However, this work was implemented by Defence Construction Canada (DCC) as part of the larger DEW Line Clean-Up (DLCU) project.  As such, this work was not under the care custody or control of Nasittuq Corporation.

Annex A

Designated landfill location, with GPS coordinates at FOX-M.
FOX-M

[image: image2.png]



Annex B
Sewage outfall location, with GPS coordinates at FOX-M.
FOX-M

[image: image4.png]68.760972, 81.224566
| 4680 45'30.50°, 81°13'28.40"

Get directions: Ta here - From here
Search nearby - Save to by Maps





[image: image5.png]



Annex C
Sewage analysis for FOX-M.





PAGE  
1

