| Dr. Andrew Medeiros | Dr. Barret Kurylyk |
| --- | --- |
| Assistant Professor | Assistant Professor |
| Centre for Water Resource Studies, Dalhousie University | Centre for Water Resource Studies, Dalhousie University |
| andrew.medeiros@dal.ca | barret.kurylyk@dal.ca |

Water Resource Assessment for Coral Harbour

Water supply for Coral Harbour, Nunavut is sourced from a river-fed reservoir. There have been instances of high electrical conductivity, which raises concerns regarding saltwater intrusion from Hudson Bay. A team (Dr. Barret Kurylyk and Dr. Julia Guimond) from the Dalhousie Coastal Hydrology Lab plans to conduct a reconnaissance field campaign to characterise the river hydrodynamics and to investigate potential sources and pathways for saltwater, including via the river, the shallow subsurface, and sea spray. The team will use non-invasive instruments to investigate the interactions between the bay and the river and to assess potential subsurface salinity dynamics. Loggers will be placed along the river to measure water level, salinity, and temperature, which will provide new understanding on coastal zone mixing in the river. Geophysical instruments will be used to measure the ground electrical resistivity to map where there are saltwater (low resistivity) and freshwater (high resistivity) zones. Surveys will be taken along the coastline and the riverbanks. Water samples will be collected to transport back to Halifax to assess the water chemistry and to fingerprint salt sources. Sensors may be used to measure total dissolved solids, dissolved oxygen, and other water quality parameters in the river. Collectively, these data will reveal the tidal dynamics in the river and help identify potential mechanisms for drinking water salinization. The first trip will be in early July (e.g. 5 days in Coral Harbour), and a potential follow-up trip will likely be undertaken later in the summer (late July to early August). Results will be shared with and interpreted for the community and Territory as desired.